# Standalone Solution

# Prerequisites

## Software

Following are the software prerequisites to get started with the standalone solution:

- Windows/Linux PC

- Wi-Fi Access Point

- Minicom/TeraTerm VT Utility installed on PC (add a download link)

- EVK package

- SDK package

### Evaluation Kit (EVK)

#### Download the Evaluation Software Package (EVK)

The EVK package (Open_Web_SDK_x.y) includes various evaluation binaries and the associated programming tools for the INP301x EVB-A boards to enable quick evaluation of the INP101x modules.

Download the software package from the InnoPhase website: https://innophaseiot.com/talaria-two-modules/#product-availability.

#### EVK Package Contents Walkthrough

The EVK package contains the following folders:

- alexa_ready: Firmware images for Alexa_Ready application which can be used to understand the protocols and functionality supported on Talaria TWO.

- at: Firmware images for AT Commands application.

- Download_Tool: Talaria TWO programming tool (available for both Windows & Linux platforms)

- I-CUBE-T2-STW-lib: STM software package for STM’s NUCLEO-STM32L4A6ZG or NUCLEO-STM32L433RC-P with ready-to-run firmware examples to support quick evaluation and development of MQTT/HTTP/AWS/AZURE and HTTPS IoT Cloud applications on STM32L4 Nucleo boards with InnoPhase Talaria TWO Wi-Fi add-on boards.

Folder details are as follows:

  - Documentation: This folder includes all applications related to Talaria TWO and STMCubeL4.

  - Drivers: This folder includes BSP files for STM32L4 Nucleo boards. Refer to release notes for further details

  - Middleware: This folder includes Middleware FreeRTOS and third-Party Middleware InnoPhase-HAPI to act as a host interface for Talaria TWO Modules (INP101X) Wi-Fi devices

  - Projects: This folder includes the applications to demonstrate the INP101X features such as Wi-Fi, BLE, cloud apps on modules 32L4A6ZG-NUCLEO and 32L433RC-P-NUCLEO. Refer to release notes for further details.

  - Utilities: This folder includes Download tool GUI and the firmware binary for Talaria TWO EVB (INP301x)

- MPD: Multi-purpose Demo GUI for evaluating example applications and network protocols

### Software Development Kit

#### Download the Software Development Package (SDK)

To download the software development package, execute the following steps:

- Go to the InnoPhase website (https://innophaseiot.com/) and click on Register.

- Provide the appropriate details to register onto the InnoPhase Customer Portal.

Note: InnoPhase requires a signed MNDA and Development Tool License Agreement (DTLA) to be signed prior to granting access to the Customer Portal.

- On successfully registering to the Customer Portal, the following screen will appear:

- Navigate to the Software Tab and download the appropriate software package(s):

#### SDK Package Contents Walkthrough

The SDK software package (sdk_x.y.zip) provides software APIs with ready-to-run firmware examples to support quick evaluation and development of MQTT/HTTP/HTTPS/AWS and AZURE IoT Cloud applications with Talaria TWO.

Folder details are as follows:

- pc_tools: Programming (Download), multi-purpose demo GUI and T2_Flasher tools. These tools are available for both Windows and Linux platforms.

- binaries: Firmware images which can be used to understand the protocols and functionality supported on Talaria TWO. The eval folder consists of  firmware image for MPD and Alexa_Ready application, while the product folder consists of the firmware image for AT commands.

- apps: production ready example applications that the user can run on Talaria TWO. Following applications are available in the release package:

| Protocol | Description |
| --- | --- |
| alarm | Describes alarm functionality in Talaria TWO. |
| filesystem | Contains APIs to check and mount Talaria TWO's file system |
| fota | Demonstrates the FOTA (Firmware Over The Air) process |
| gordon | Flashing utility using UART interface |
| gordon-jtag | Flashing utility using JTAG / SWD interface |
| helloworld | Basic helloworld application |
| iot_aws | Demonstrates using Talaria TWO board and the SDK with Amazon Web Services (AWS) IoT |
| iot_azure | Readme file containing the link to IoT_Azure application note present in Github |
| iperf3 | iperf3 server application on Talaria TWO |
| ssbl | Demonstrates running and switching between multiple applications on Talaria TWO using SSBL |
| stw | Serial to Wi-Fi application over HIO |
| stw_multi_proto | Serial to Wi-Fi application used in hosted mode of operation |

- examples: Demo/sample applications that the user can run on Talaria TWO firmware. Following examples are available:

| Protocol | Description |
| --- | --- |
| adc | Example for using the Analog to Digital Convertor (ADC) peripheral of Talaria TWO modules |
| at_custom_cmd | Demonstrates custom AT commands which the user can use apart from the standard commands |
| ble_beacons | Example codes describing the basic concept of Eddystone Beacon - Eddystone UID - Eddystone URL - Eddystone TLM |
| ble_wifi_bridge | Example code for receiving a text message from a connected BLE client and publishing it to a CloudMQTT broker |
| chip_monitor | Describes the application for fetching the changes in the values of device core temperature, Voltage of VBAT, external ADC and estimated current consumption of Talaria TWO device |
| crash_handling | Example code for using the crash handler API to handle and debug error cases |
| gpio | Example codes for using the interface for GPIO |
| http_client | Example codes for using HTTP client APIs to connect to HTTP servers in secured (HTTPS) and non-secured way |
| i2c | Demonstrates usage of I2C on Talaria TWO |
| i2s_audio | Example codes for using I2S peripheral of INP1010/INP1011/INP1012/INP1013 Talaria TWO modules for playing audio |
| ifttt | Example application for using Talaria TWO with IFTTT |
| innoos_memory_mgmt | Example code for basic memory management and error handling |
| innoos_msg_q | Example codes describing the use of message queue APIs |
| innoos_threads_semaphores | Example codes describing the use of threads and semaphore APIs |
| innoos_timers_callouts | Example codes describing the use of Timers and Callout APIs |
| innoos_work_q | Example codes describing the use of Work Queue APIs |
| lp_scan | Demonstrates the basics of the Low Power Wi-Fi scan feature in InnoOS™ |
| lp_uart | Example codes describing the use of UART APIs |
| mdns | Demonstrates using the mDNS APIs provided by the mDNS module |
| mqtt | Example codes for using the publish/subscribe operation of MQTT in both secured and non-secured modes |
| prov | A demo Provisioning application using BLE for provisioning AP credentials at Talaria TWO from a mobile application |
| pwm | Demonstrates usage of Pulse Width Modulation peripheral of INP1010/INP1011/INP1012/INP1013 Talaria TWO modules |
| radio_module_params | Describes using the Radio and Module parameters on the Talaria TWO module |
| sleep_enable_disable | Demonstrates the basics of sleep management in InnoOS™ (os_suspend_enable() and os_suspend_disable() functions) |
| socket_wakeup | Demonstrates the basics of sleep management in InnoOS™ (Talaria TWO wake-up from sleep mode) |
| spi | Demonstrates the usage of the Software SPI Master (SSM) on Talaria TWO |
| spi_flash | Example codes describing using of SPI Flash APIs for Talaria TWO EVK |
| un-assoc | Example codes describing Wi-Fi un-associated mode transmission APIs available in the SDK, call-back events, notifications and associated data structures |
| using_ble | Introduction to BLE APIs through code samples consisting of a server and client application |
| using_filesystem | Demonstrates using the filesystem APIs to show case the filesystem functionalities on the Talaria TWO EVK |
| using_hio | Demonstrates the fundamentals of developing HIO based application on both host and the Talaria TWO EVK |
| using_sntp | Demonstrates fetching time from NTP server using SNTP |
| using_wifi | Example codes describing the Wi-Fi connection manager APIs |
| watchdog_timer | Demonstrates managing Talaria TWO watchdog timer using the functions provided by the watchdog driver |
| wcm_multi_ap | Demonstrates the application example of Wi-Fi Connection with Multi-Access Point (WCM_MULTI_AP) available in the SDK |
| wcm_pm | Demonstrates the Wi-Fi Connection Manager power management APIs |
| websocket | Demonstrates using the WebSocket client APIs provided by the WebSocket module |

- conf: Make and debugger configuration files. These files include Id and linker scripts and make file rules used for firmware configuration.

- doc:

The doc folder contains the following sub-folders:

  - reference_guides

    - api_reference_guide: API reference guide describes the programmers APIs.

    - bootargs_reference_guide: Bootargs reference guide describes the use of Talaria TWO boot arguments (bootargs) in different scenarios using different feature and protocols.

    - cli_reference_guide: CLI reference guide serves as a reference guide for Talaria TWO CLI commands, its usage, use case or examples in different scenarios supported by Talaria TWO modules.

  - user guides.

    - ug_evb_a: Talaria TWO Evaluation Board an overview of the evaluation board explaining its key features and functions

    - ug_eclipse_setup_windows: Eclipse setup in Windows describes developing an application using Eclipse and Talaria TWO SDK

    - ug_eclipse_setup_linux: Eclipse setup in Linux describes the procedure to build and debug an application on Eclipse IDE using Talaria TWO SDK

    - ug_env_setup_linux: Environment set-up with Talaria TWO for Linux describes setting up the development environment for Talaria TWOTM SDK on an Ubuntu VirtualBox with a Windows 10 host

    - ug_wsl: Windows Subsystem for Linux describes developing an application using Windows Subsystem for Linux (WSL) and Talaria TWO SDK

    - ug_coredump_generation_and_anaysis: Coredump generation and analysis describes generating a coredump file used for analyzing the cause of the crash

    - ug_debugging_using_gdb: Debugging using GDB describes the procedure for debugging the applications using GDB to work with OpenOCD

    - porting_guide_freertos_to_innoos: Porting guide describes the procedure to port FreeRTOS code to InnoOS

    - porting_guide_zephyros_to_innoos: Porting guide describes porting of the ZephyrOS code to InnoOS

    - ug_firmware_sdk: Firmware SDK user guide describes developing applications for the Talaria TWO device

    - ug_programming_using_black_magic_board: Black magic board user guide describes the Black Magic Programmer Board which provides a programming and debugging interface for Talaria TWO modules

    - ug_programming_using_INP3000: The INP3000 programming user guide describes the INP3000 programmer board which provides a programming interface for Talaria TWO modules

    - ug_visual_studio_setup_windows: Visual Studio Setup in Windows user guide describes the procedure to build and debug an application on Visual Studio Code Editor in Windows using Talaria TWO SDK

- components: Files which serve as common components which all applications can make use of. It acts as a library, where an application like HTTP, SNTP, etc., can link to this library and directly use the function.  The following protocols are available in the components folder serving as a library for applications:

  - alarm

  - checksum

  - fota

  - http

  - json

  - mdns

  - mqtt

  - out

  - prov

  - sntp

  - ssl_wrap

  - utils

  - websocket

- include: SDK include files (.h files) for all applications/examples in the release package.

- lib: SDK library files which can be used by all applications/examples.

- script: Helper scripts/utilities used to achieve multiple functionalities like program, read, write and so on.

- tools: Certain tools, supporting files and scripts used for development on Talaria TWO. This directory further contains the following three directories:

  - fletcher32 - tool used to create checksum files for checking the integrity of the configuration files. Currently part.json, boot.json and fota_config.json files’ integrity is checked using the checksum.

  - mklittlefs - tool used to create the file system image.

  - partition_files - flash partition files used with SSBL setup (ssbl_part_table.json) and without SSBL setup (standard_part_table.json).

- root_fs: Basic files that need to be present in root_fs image as required for SSBL. It also contains the script to build the root_fs.

- solutions: Contains pre-built binaries, source code for custom applications, library, json file and root files for Dual-Stack solution.