# Programming Talaria TWO over SWD

The INP3000 Programmer board version 4.0 has the SWD interface to program Talaria TWO modules.

Install the appropriate dependencies for programming over JTAG.

## In Windows

- Open command prompt and reset the device in a boot loader mode by executing the following command from the SDK directory:

.\script\reset.py evk42_bl

Figure 18: SWD- Resetting the device in boot loader mode- Console output (Windows)

- In the same terminal start OpenOCD by executing the following command from the SDK directory:

openocd -s .\conf -f ftdi_swd.cfg -f t2_swd.cfg

The following output is observed on the command prompt console:

Figure 19: SWD - Starting OpenOCD – console output – Windows

- In a separate command prompt window, execute the following command from the SDK directory:

.\pc_tools\T2_Flasher\bin\T2_Flasher_Windows.exe --operation=write_ptable .\<path to the standard_part_table.json file available in sdk_x.y/tools/partition_files directory>

The following output is observed after flashing the default partition table:

Figure 20: SWD - Flashing the default partition table – console output

- In the same terminal, execute the following command from the SDK directory to flash the data image (data.img).

Note: This step is needed only if the application uses any certificates.

.\pc_tools\T2_Flasher\bin\T2_Flasher_Windows.exe --operation=write_part --partition=DATA .\<path to the generated data image>\data.img

The following output is observed after flashing the data image:

Figure 21: SWD - Flashing the data image - console output

  - To generate the application image, refer section: In Windows. In the same terminal, execute the following command from the SDK directory to flash the application image:

.\pc_tools\T2_Flasher\bin\T2_Flasher_Windows.exe --operation=write_part --partition=BOOT .\<path to the generated application image>\app.img

The following output is observed after flashing the application image:

Figure 22: SWD - Flashing the application image - console output

  - In the same terminal, flash the VM image of the application by executing the following command:

.\pc_tools\T2_Flasher\bin\T2_Flasher_Windows.exe --operation=write_part --partition=VIRT .\<path to the generated application image.vm>\app.img.vm

The following output is observed after flashing the application’s VM image:

Figure 23: SWD - Flashing application's VM image - console output

The application is successfully flashed over SWD. Now, OpenOCD needs to be terminated before debugging using Eclipse. Close all the command prompt windows to terminate OpenOCD.

- On INP3000 version 4.0, the console output can be seen using the Download Tool by clicking the reset button.

For more details on using the Download tool, refer document: UG_Download_Tool.pdf (path: sdk_x.y\pc_tools\Download_Tool\doc).

Figure 24: INP3000 version 4.0 - Download tool console output – Windows

## In Linux

- Open command prompt and reset the device in boot loader mode by executing the following command from the SDK directory:

./script/reset.py evk42_bl

Figure 25: SWD - Resetting the device in boot loader mode- console output (Windows)

- Open the command prompt and start OpenOCD by executing the following command from the SDK directory:

openocd -s ./conf -f ftdi_swd.cfg -f t2_swd.cfg

The following output is seen on command prompt console:

Figure 26: SWD - Starting OpenOCD – console output

- Flash the default partition table by executing the following command:

./pc_tools/T2_Flasher/bin/T2_Flasher_Linux --operation=write_ptable .\<path to the standard_part_table.json file available in sdk_x.y/tools/partition_files directory>

Following output is observed after flashing the default partition table:

Figure 27: SWD - Flashing the default partition table – console output

- In the same terminal, execute the following command from the SDK directory to flash the data image (data.img).

Note: This step is needed only if the application uses any certificates.

./pc_tools/T2_Flasher/bin/T2_Flasher_Linux --operation=write_part --partition=DATA ./<path to the generated data image>/data.img

The following output is observed after flashing the data image:

Figure 28: SWD - Flashing the data image - console output

- In a separate command prompt window, execute the following command from the SDK directory to flash the application image:

./pc_tools/T2_Flasher/bin/T2_Flasher_Linux --operation=write_part --partition=BOOT ./<path to the generated application image>/app.img

Following output is observed after flashing the application image:

Figure 29: SWD - Flashing the application image - console output

- In the same terminal, flash the VM image of the application by executing the following command:

./pc_tools/T2_Flasher/bin/T2_Flasher_Linux –operation=write_part –partition=VIRT ./<path to the generated application image.vm>./app.img.vm

Following output is observed after flashing the application’s VM image:

Figure 30: SWD - Flashing application's VM image – console output

The application is successfully flashed over SWD. Now, OpenOCD needs to be terminated before debugging using Eclipse. Close all the command prompt windows to terminate OpenOCD.

- On INP3000 version 4.0, the console output can be seen using the Download Tool by clicking on a reset button.

For more details on using the Download tool, refer document: UG_Download_Tool.pdf (path: sdk_x.y\pc_tools\Download_Tool\doc).

Figure 31: INP3000 version 4.0 - Download tool console output – Linux