Following are the software prerequisites to get started:

- Windows/Linux PC

- Wi-Fi Access Point

- Minicom/TeraTerm VT Utility installed on PC (add a download link)

- SDK package

### Software Development Kit

#### Download the Software Development Package (SDK)

To download the software development package, execute the following steps:

- Go to the InnoPhase website (https://innophaseiot.com/) and click on Register.

- Provide the appropriate details to register onto the InnoPhase Customer Portal.

Note: InnoPhase requires a signed MNDA and Development Tool License Agreement (DTLA) to be signed prior to granting access to the Customer Portal.

- On successfully registering to the Customer Portal, the following screen will appear:

- Navigate to the Software Tab and download the appropriate software package(s):

#### SDK Package Contents Walkthrough

The FreeRTOS SDK software package (sdk_x.y.zip) provides software APIs with ready-to-run firmware examples to support quick evaluation and development of MQTT/HTTP/HTTPS/AWS and AZURE IoT Cloud applications with Talaria TWO.

Folder details are as follows:

- apps: production ready example applications that the user can run on Talaria TWO. Following applications are available in the release package:

| Protocol | Description |
| --- | --- |
| alarm | Describes alarm functionality in Talaria TWO |
| helloworld | Basic helloworld application |
| ssbl | Demonstrates running and switching between multiple applications on Talaria TWO using SSBL |
| stw_multi_proto | Serial to Wi-Fi application used in hosted mode of operation |
| gordon.elf | Flashing utility using UART interface |

Table 1: Application in the release package

- binaries: The product folder consists of firmware image for AT commands.

- bins: Firmware images which can be used to understand the protocols and functionality supported on Talaria TWO.

- build: Make configuration files. These files include linker scripts and make file rules used for firmware configuration.

- components: Files which serve as common components which all applications can make use of. It acts as a library, where an application like HTTP, SNTP, etc., can link to this library and directly use the function. The following protocols are available in the components folder serving as a library for applications:

  - alarm

  - checksum

  - cipher

  - device_ifc

  - fota

  - http

  - json

  - mdns

  - mqtt

  - out

  - prov

  - secureboot

  - sntp

  - ssl_wrap

  - utils

  - websocket

- conf: Debugger configuration files.

- doc:

The doc folder contains the following sub-folders:

  - reference_guides

    - api_reference_guide: API reference guide describes the programmers APIs.

    - bootargs_reference_guide: Bootargs reference guide describes the use of Talaria TWO boot arguments (bootargs) in different scenarios using different feature and protocols.

    - cli_reference_guide: CLI reference guide serves as a reference guide for Talaria TWO CLI commands, its usage, use case or examples in different scenarios supported by Talaria TWO modules.

  - user_guides

    - ug_evb_a: Talaria TWO Evaluation Board an overview of the evaluation board explaining its key features and functions

    - ug_eclipse_setup_windows: Eclipse setup in Windows describes developing an application using Eclipse and Talaria TWO SDK

    - ug_eclipse_setup_linux: Eclipse setup in Linux describes the procedure to build and debug an application on Eclipse IDE using Talaria TWO SDK

    - ug_env_setup_linux: Environment set-up with Talaria TWO for Linux describes setting up the development environment for Talaria TWOTM SDK on an Ubuntu VirtualBox with a Windows 10 host

    - ug_wsl: Windows Subsystem for Linux describes developing an application using Windows Subsystem for Linux (WSL) and Talaria TWO SDK

    - ug_coredump_generation_and_anaysis: Coredump generation and analysis describes generating a coredump file used for analyzing the cause of the crash.

    - ug_debugging_using_gdb: Debugging using GDB describes the procedure for debugging the applications using GDB to work with OpenOCD

    - porting_guide_innoos_to_freeRTOS: Porting guide describes the procedure to port InnoOS code to FreeRTOS.

    - ug_firmware_sdk: Firmware SDK user guide describes developing applications for the Talaria TWO device.

    - ug_memory_mapping: Memory mapping describes the flash memory mapping of standard applications with details on the flash memory layout.

- examples: Demo/sample applications that the user can run on Talaria TWO firmware. Following examples are available:

| Protocol | Description |
| --- | --- |
| at_custom_cmd | Demonstrates custom AT commands which the user can use apart from the standard commands |
| ble_beacons | Example codes describing the basic concept of Eddystone Beacon - Eddystone UID - Eddystone URL - Eddystone TLM |
| ble_wifi_bridge | Example code for receiving a text message from a connected BLE client and publishing it to a CloudMQTT broker |
| chip_monitor | Describes the application for fetching the changes in the values of device core temperature, Voltage of VBAT, external ADC and estimated current consumption of Talaria TWO device |
| crash_handling | Example code for using the crash handler API to handle and debug error cases |
| gpio | Example codes to use the GPIO interface. the interface for GPIO |
| http_client | Example codes for using HTTP client APIs to connect to HTTP servers in secured (HTTPS) and non-secured way |
| i2c | Demonstrates usage of I2C on Talaria TWO |
| ifttt | Example application for using Talaria TWO with IFTTT |
| lp_scan | Demonstrates the basics of the Low Power Wi-Fi scan feature |
| mdns | Demonstrates using the mDNS APIs provided by the mDNS module |
| mqtt | Example codes for using the publish/subscribe operation of MQTT in both secured and non-secured modes |
| prov | A demo Provisioning application using BLE for provisioning AP credentials at Talaria TWO from a mobile application |
| secure_files | Example application for reading and writing encrypted files from/to the filesystem |
| unassoc | Example codes describing Wi-Fi un-associated mode transmission APIs available in the SDK, call-back events, notifications and associated data structures |
| using_ble | Introduction to BLE APIs through code samples consisting of a server and client application |
| using_filesystem | Demonstrates using the filesystem APIs to show case the filesystem functionalities on the Talaria TWO EVK |
| using_sntp | Demonstrates fetching time from NTP server using SNTP |
| using_wifi | Example codes describing the Wi-Fi connection manager APIs |
| watchdog_timer | Demonstrates managing Talaria TWO watchdog timer using the functions provided by the watchdog driver |
| wcm_pm | Demonstrates the Wi-Fi Connection Manager power management APIs |
| websocket | Demonstrates using the WebSocket client APIs provided by the WebSocket module |

Table 2: Example applications

- FreeRTOS-Kernel: FreeRTOS include files.

- include: SDK include files (.h files) for all applications/examples in the release package.

- libs: SDK library files which can be used by all applications/examples.

- pc_tools: Programming (Download) tool and Multi-Purpose Demo tool (available for both Windows and Linux platforms).

- root_fs: Contains the filesystem image.

- script: Helper scripts/utilities used to achieve multiple functionalities like program, read, write and so on.

- tools: Certain tools, supporting files and scripts are used for development on Talaria TWO. This directory further contains the following directories:

  - fcc_test – tool used for Wi-Fi/BLE testing and to set up required test parameters.

  - fletcher32 - tool used to create checksum files for checking the integrity of the configuration files. Currently part.json, boot.json and fota_config.json files’ integrity is checked using the checksum.

  - mklittlefs - tool used to create the file system image.

  - partition_files - flash partition files used with SSBL setup (ssbl_part_table.json) and without SSBL setup (standard_part_table.json).