This section serves as a reference guide for Talaria TWO CLI commands, its usage, use case or examples in different scenarios supported by Talaria TWO modules.

# Introduction

The subsequent sections introduce the use of commands in the CLI environment to access the Talaria TWO (T2) module and configure the device to perform various development, automation and testing tasks.

These commands run Python scripts from the host machine (Windows or Linux), in conjunction with the application software that are loaded to and are running in the Talaria TWO device. The script files and the Talaria TWO reference/example applications are provided in the Talaria TWO SDK release package in the following folders:

- freertos_sdk/apps: Talaria TWO applications that are the core and basic release applications.

- freertos_sdk/examples: Collection of Talaria TWO software apps along with components for Talaria TWO software development. They are organized in various technology areas and implementation examples, together with the projects and make files.

- freertos_sdk/script: Python scripts/utilities to configure and program the device.

- Other folders.

# Primary Scripts for CLI Operations

Written in the Python language, three script files for executing CLI operations are provided in the SDK for the user’s convenience that can be used for evaluation, testing, and development of Talaria TWO based systems.

- sdk/script/boot.py

- sdk/script/flash.py

- sdk/script/storage.py

Based on the arguments provided command line, the scripts performs various functions.

## boot.py

Following are the basic functionalities of boot.py:

- Load app to memory (RAM) on Talaria TWO module:

This script reads the application (in ELF format) provided in command line, along with the arguments to load the app to Talaria TWO in its memory, via the host and device interface. USB is used only in the evaluation, development or testing phase.

- Write app to flash on Talaria TWO module:

This script reads the application (in ELF format) provided in command line, along with the arguments to write the app to Talaria TWO in its flash, with the default handling by the script, via the host and device interface. USB is used only in the evaluation, development or testing phase.

- Create .img file from ELF file

This script reads the name of the app file (in ELF format) provided in command line to generate the .img file for flash download.

Note: Refer section Using boot.py for more details.

## flash.py

Script to read and write to the flash, which requires gordon.elf application.

This section provides details on script usage and prerequisites.

Note: Refer section Using flash.py for more details.

Prerequisite/precondition to run flash.py:

- Program Talaria TWO with the Gordon app (gordon.elf)

- gordon.elf application is available from the following folder:  freertos_sdk_x.y/apps/gordon.elf.

- Gordon is an application that enables access to Talaria TWO's flash over Host I/O, including partitioning and filesystem access and raw reads/writes to flash.

- Boot.py script is used to program the gordon.elf onto Talaria TWO. Execute the following command to accomplish the same:

./script/boot.py [device] <path>/gordon.elf

Load app .img file to flash on Talaria TWO module:

This script reads the name of the app file (in .img format) provided in command line, along with the arguments to write the app to Talaria TWO in its flash with the write command and user-specified location (in hex), via the host and device interface (USB/Serial).

Create image file (.img) from the ELF file of an application:

./script/boot.py –output <output_path>/app.img <elf_path>/app.elf

Read/Write Partition Table on module:

This script reads from/writes the partition table on the device (in the json file).

## storage.py

Storage file access utility. Following are the basic file system access functionalities of storage.py:

- Display the file system contents

- Read file from the device to store in the location on host

- Write file from the host to the device

- Write the network config file (.json file) into Talaria TWO file system to configure L2 parameters such as SSID, passphrase, BSSID, security type and certificates for EAP authentication

Refer section Storage Data & File: Read/Write for the details.

# Device Interface on Host Machine

CLI commands are supported on both Linux and Windows OS. However, there are some limitations on the same functionality in the Windows environment, which are highlighted in the respective sections.

## Device Port Identification on Linux OS

Generally, the host interface connected to the device takes the form of the device USB ports on Linux OS, /dev/ttyUSBn. Once connected to Talaria TWO EVB, the host detects 4 USB ports for the connected device.

Type in the Ls command in Linux to list the USB ports:

$ ls /dev/ttyUSB*

Figure 1: USB ports

From the ports listed, identify the following ports:

- /dev/ttyUSB2: Connected to the peripheral UART of Talaria TWO module

- /dev/ttyUSB3: Connected to the console port (GPIO17) which is unidirectional UART from the Talaria TWO module. It operates at a high baud rate 2457600 (default), used for debug prints.

With this information available, all CLI commands should use the following argument for the peripheral port to the device in the command line:

./script/boot.py --device /dev/ttyUSB2 --reset=evk42_bl ./apps/hello_world/bin/hello_world.elf

This loads the helloworld app provided in SDK to Talaria TWO memory.

Launch miniterm, a small terminal application, to open the /dev/ttyUSB3 device port.

$ miniterm /dev/ttyUSB3 2457600

When an application runs on the device, it prints out debug messages through the console port.

Figure 2: Debug messages displayed through console port

If there are multiple Talaria TWO devices where each of them has their own console port connected to the host via the USB interface, it will be sufficient to use the respective /dev/ttyUSBn with the command.

For instance, two Talaria TWO devices with USB connected.

Figure 3: 2 Talaria TWO devices with USB connected

Figure 4 and Figure 5 show that the miniterm app is run on two separate terminals using respective device ports to communicate with the respective device.

Figure 4: /dev/ttyUSB3

Figure 5: /dev/ttyUSB7

## Device Port Identification on Windows OS

On a Windows platform, some control functionalities associated with the CLI device-reset feature requires additional support provided by installation of the libusbK library.

There are two ways for a Windows host to interface with the device EVB connected to the host via USB interface:

- A Windows system with the libusbK installed (recommend)

The library can be installed with the Zadig tool:  https://zadig.akeo.ie/

- A Windows system without the libusbK installed

This is the default configuration when the EVB with USB is detected by Windows. It finds a generic, default USB/Serial driver to handle the device interface. In this setup, not all functionalities offered by the script tools are available.

Depending on the environment where the libusbK is installed or not, CLI use can be described as follows:

On Windows Platform with libusbK installed (recommended)

On this serial communication setup, Windows host is communicating with the device using the libusbK driver.

The libusbK’s installation and use by the host can be checked from the Windows Device Manager display.

Figure 6: libusbK driver installation

If there are multiple Talaria TWO devices connected, the Windows Device Manager appears:

Figure 7: Multiple Talaria TWO devices connected

With this setup in place, all CLI commands should use the following argument for the input port to the device in command line:

ftdi://ftdi:4232/3

where,

- 4232: Device ID for the FTDI 4232H device on EVK

- 3: FTDI UART interface ID for the input/programming on EVK

With this information available, all CLI commands should use the following argument for the peripheral port to the device in the command line:

python script\boot.py --device ftdi://ftdi:4232/3 --reset=evk42_bl apps/hello_world/bin/hello_world.elf

Figure 8: Flashing to Talaria TWO

This command loads the helloworld app provided in SDK to Talaria TWO memory.

If there are multiple Talaria TWO devices where each of them has their own peripheral port connected to the host via the USB interface, the device specifier must be expanded with the serial ID of the device in the command line. This is accomplished with the keyword --SN. Execute the following steps:

- Append to the device port with the following string:

ftdi://ftdi:4232:<device_serial_id>/3, which is followed by: --SN <device_serial_id>

- Peripheral port to the device in the command line:

python script\boot.py --device ftdi://ftdi:4232:10110001/3 --reset=evk42_bl --SN 10110001 apps/hello_world/bin/hello_world.elf

Figure 9: Expanding the device specifier

<device_serial_id>:

The hyphen ‘-‘ is a valid character in a serial ID of a device,  if present. In other words, a serial ID of 11010391 is considered different than 1101-0391.

Example of a device with serial ID 00166d4:

python script\boot.py --device ftdi://ftdi:4232:0016d4/3 --reset=evk42_bl --SN 0016d4 apps/hello_world/bin/hello_world.elf

Figure 10: Device with serial ID 0016d4

On Windows Platform using COM port (without the libusbK installed)

On this serial communication environment, Windows host is communicating with the device using the standard (default) USB/Serial driver.

Figure 11: Standard (default) USB/serial driver

In the listed ports, identify the following ports:

- COM6: Connected to the peripheral UART of Talaria TWO module

- COM7: Connected to the console port (GPIO17) which is unidirectional UART from Talaria TWO module and operates at a high baud rate 2457600 (default), used for debug print

With this setup in place, all CLI commands can use the following argument for the input port to the device in the command line:

python script\boot.py --device COM6 apps\gordon.elf

This command loads the gordon app provided in SDK to Talaria TWO memory.

# CLI Command Syntax and List Parameters

All commands with arguments are entered in a space-separated fashion.

Positional and Optional Arguments

Arguments should be passed on to the app to be loaded into the Talaria TWO device with the script. There are two different types of arguments:

- Positional Arguments

Positional arguments must be included in the right order. Their positions in the sequence should be correct.

- Optional Arguments

Optional arguments are optional. Defaults will be used when the arguments are not specified in the command line.

## Using boot.py

- Command syntax to run boot.py

<path_script>/boot.py [device] <path_app_elf>/app.elf [boot_args]

- where,

  - <path_script>: Path to the script file. Default: Current folder

  - [device]: Communication device ID with path.

    - If there is only one Talaria TWO device with USB interface connected:

    - Examples: COMn(Windows), /dev/ttyUSBn(Linux), ftdi://ftdi:xxxxxx(Windows libusbK installed).

    - If there are multiple Talaria TWO devices where each of them has their own peripheral port connected to the host via the USB interface:

    - The device specifier must be expanded with the serial ID of the device in the command line.  This is accomplished with the keyword --SN.  Execute the following:

    - Command for device ID to the device port as shown above with the string is as follows:

./script/boot.py --device /dev/ttyUSB2 --SN 15100010 --reset=evk42_bl bins/iperf3.elf ssid=InnoPhase passphrase=Inno@9070

Refer section Device Interface on Host Machine for information on multiple device connection on the host machine (Linux and Windows).

Default: Windows-none, Linux-/dev/ttyUSB2

- <path_app_elf>: Path to the app ELF file

- [boot_args]: Bootargs. Examples: ssid=InnoPhase, passphrase=Inno@9070

Examples:

- If there is only one Talaria TWO device with USB interface connected:

Note: A single line command ./script/boot.py --device /dev/ttyUSB2 --reset=evk42_bl bins/iperf3.elf ssid=InnoPhase passphrase=Inno@9070

Figure 12: Single Talaria TWO device with USB interface

- If there are multiple Talaria TWO devices connected:

Note: A single line command ./script/boot.py --device /dev/ttyUSB2 --SN 0016d4 --reset=evk42_bl bins/iperf3.elf ssid=Lucy passphrase=Password@321

Figure 13: Multiple Talaria TWO devices connected

List Parameters (Manual in Script)

Type boot.py --help to bring up the manual of the boot.py command list:

Figure 14: boot.py - command list

## Using flash.py

- Command syntax to run flash.py

<path_script>/flash.py [device] <pos_arg> [boot_args]

where:

  - <path_script>: Path to the script file. Default: Current folder

  - [device]: Communication device ID with path.

For example: COMn(Windows), /dev/ttyUSBn(Linux), ftdi://ftdi:xxxxxx(Windows libusbK installed).

Default: Windows-none, Linux-/dev/ttyUSB2

  - <pos_arg>: Positional argument

  - [boot_args]: Bootargs

Example:

./script/flash.py --device /dev/ttyUSB2 to_json ./brd_0183_part.json

- List Parameters (Manual in Script)

Type flash.py --help to bring up the manual of the flash.py command list:

Figure 15: flash.py - command list

## Using storage.py

- Command syntax to run storage.py

<path_script>/storage.py [device] <pos_arg> [boot_args]

where:

  - <path_script>: Path to the script file. Default: Current folder

  - [device]: Communication device ID with path.

For example: COMn, /dev/ttyUSBn, ftdi://ftdi:xxxxxx. Default: Windows-none, Linux-/dev/ttyUSB2

  - <pos_arg>: Positional argument

  - [boot_args]: Bootargs

Example:

Connect to the Access Point configured in your nprofile.json as

./script/storage.py write nprofile.json /data/ nprofile.json

- List Parameters (Manual in Script)

Type storage.py --help to bring up the manual of the storage.py command list:

Figure 16: storage.py command script

# Use Cases

## Linux Platform

### Reset Device

- Normal Device Reset

Reset the device.

Command syntax:

./script/boot.py [device] --reset=evk42_bl

Figure 17: Normal device reset - terminal output

- Device Reset with Inhibiting-flash-boot (Boot loader mode)

Reset the device with the inhibiting-flash-boot feature.

If the device has app code stored in flash from which the device will boot, this special reset command is required which will allow flashing of a new app code.

Command syntax:

./script/boot.py [device] --reset=evk42_bl

Figure 18: inhibiting-flash-boot - terminal output

This reset will trigger Talaria TWO to abort from device port link (expected).

Figure 19: Abort from device port link

Launch miniterm to resume the device port connection.

Figure 20: Resume device port connection

- Reset with Inhibiting-flash-boot with booting Gordon

Reset the device with inhibiting-flash-boot and boot the gordon app.

This combination of commands is for the host to obtain access to the storage on Talaria TWO and operate read/write data.

Command syntax:

./script/boot.py [device] --reset=evk42_bl apps/gordon.elf

Figure 21: Inhibiting-flash-boot with booting Gordon - terminal output

### Program App in Memory

Program app in Talaria TWO memory.

Command syntax:

./script/boot.py [device] --reset=evk42 <path>/app.elf [boot_args list]

Figure 22: Programming app in Talaria TWO memory - terminal output

### Erase Image or Program New Image in Flash

- Erase Image in Flash

The following command erases the app code in flash:

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Flash empty.img

Command syntax:

./script/flash.py [device] write 0x1000 <path>/empty.img

Figure 23: Flash empty.img

Button reset to confirm the erase operation is a success.

Figure 24: Erase operation success

- Program New Image in Flash

Command syntax:

./script/boot.py [device] --reset=evk42_bl --flash=all <path>/app.elf [boot_args_list]

Figure 25: Program new image to flash

Button reset to confirm the flash operation a success.

Figure 26: Flash operation success

### Storage Data & File: Read/Write

- Read Partition Table from Device (to_json)

Reading file from device involves storage access. Hence, running of Gordon on the device is necessary.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the flash.py with the command: to_json

Command syntax:

./script/flash.py [device] to_json <user_specify_path>/<user_spefcify_name>.json

Figure 27: Inhibiting-flash-boot and load gordon.elf

Display of the board_356.json file

Figure 28: Display of the board_356.json file

- Write Partition Table in Device (from_json)

Writing file to device involves storage access. Hence, running of Gordon on the device is necessary.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the flash.py with the command: from_json

Command syntax:

./script/boot.py [device] --reset=evk42_bl <path>/gordon.elf ./script/flash.py [device] from_json <user_specify_path>/<user_spefcify_name>.json

- Show File System Contents

Display the file system contents.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the storage.py with the command: tree

Command syntax:

./script/storage.py tree

Figure 29: Show file system contents

- Read File and Store to Location on Host

Read file from the file system and store it in host.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the storage.py with the command: read

Command syntax:

./script/storage.py read <path_read>/<file_name_read> --output <path_write>/<file_name_write>

The file is created:

Figure 30: File creation

- Write File to Device

Write file from host to the device in file system.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the storage.py with the command: write

Command syntax:

./script/storage.py write <path_read>/<file_name_read> <path_write>/file_name_write>

Figure 31: Running storage.py - write

Use ./script/storage.py tree command to check the new file on Talaria TWO file system:

Figure 32: Check new file on Talaria TWO file system

Example: Load certificates

- $ ./script/storage.py write rootCACert.pem /data/ca.pem

- $ ./script/storage.py write client_cert.pem /data/client.pem

- $ ./script/storage.py write client_key.pem /data/client.key

### Network Profile

The Wi-Fi network profile stores the configuration of the Wi-Fi network with parameters such as SSID, passphrase, BSSID, security type etc. Network profile can be created from the JSON data.

JSON data can be stored in either:

- The filesystem: Build a network profile from a file in Talaria TWO filesystem given by the path in Talaria TWO filesystem

or

- A string: JSON string which describes and parses the network profile

Characteristics of creating JSON network profile:

- All network profiles MUST start with { and end with }.

- The network profile contents use the form ("key": "value") pairs.

- All the ("key": "value") pairs MUST be followed by a comma (,) EXCEPT the last row.

- There is NO COMMA at the end of the network profile.

- Mandatory fields MUST always be present, otherwise, parsing the network profile will fail.

- If a passphrase contains double quotes, for example: "my_secret", a backslash MUST preceded the double quotes, i.e., "passphrase": "\"my_secret\"".

Note:

To avoid recomputing the PMK for each connection (WPA/WPA2-PSK), it can be stored in Talaria TWO’s flash. This can be done by a pmk_path key and the location of PMK file in Talaria TWO flash. There are two new keys to make it easier in creating network profiles:

- security_type key which replaces the old np_type in network profiles and can have the following values depending the type of the network:

  - open

  - wpa_psk or wpa2_psk

  - wpa3_psk

  - wpa23_psk

  - enterprise

- auth_type key which can ONLY be used when configuring enterprise network profiles and describes the authentication protocol. Values for auth_type can be:

  - eap_psk

  - eap_tls

  - eap_peap

Following are some examples of JSON data specifying different network profiles:

- Open security:

{ “security_type”: “open” “ssid”: “my_ap_ssid” }

Profile fields:

  - Mandatory – security_type, ssid

  - Optional – bssid

- WPA/WPA2-PSK:

{ "security_type": "wpa23_psk", "ssid": "my_wpa2_ap_ssid", "passphrase": "my_wpa2_passphrase" “pmk_path”: “sys/pmkey_fname.data” }

Profile fields:

  - Mandatory – security_type, ssid, passphrase

  - Optional – bssid, pmk_path

- WPA/WPA2-PSK specifying one specific AP among several with same SSID. BSSID is the APs mac address:

{ "security_type": "wpa2_psk", "ssid": "my_wpa2_ap_ssid", "passphrase": "my_wpa2_passphrase", "bssid": "my_ap_bssid" “pmk_path”: “/data/pmkey.data” }

Profile fields:

  - Mandatory – security_type, ssid, passphrase

  - Optional – bssid, pmk_path

- WPA3-PSK (SAE) only:

{ "security_type": "wpa3_psk", "ssid": "my_wpa3_ap_ssid", "sae_password": "my_sae_password" }

Profile fields

  - Mandatory: security_type, ssid, sae_password

  - Optional : sae_password_id, bssid

- WPA/WPA2-PSK and WPA3-PSK. If WPA3 is available, it will be given a higher priority:

{ "security_type": "wpa23_psk", "ssid": "my_wpa2_wpa3_ap_ssid", "sae_password": "my_sae_password" "passphrase": "my_wpa2_passphrase" }

Profile fields

  - Mandatory: security_type, ssid, sae_password, passphrase

  - Optional : bssid

- EAP-PSK Enterprise security:

{ "security_type": "enterprise", "auth_type": "eap_psk", "ssid": "my_eap_psk_ap_ssid", "passphrase": "my_eap_psk_passphrase", "identity": "my_eap_identity" } Profile fields Mandatory: security_type, auth_type, ssid, passphrase, identity Optional : bssid }

- EAP-TLS Enterprise security.

Note: Storing the certificate and key files on flash is not part of the network profile functionality.

{ "security_type": "enterprise", "auth_type": "eap_tls", "ssid": "my_eap_tls_ap_ssid", "identity": "my_eap_tls_identity", "ca_path": "/data/ca.pem", "cert_path": "/data/client.pem", "pkey_path": "/data/client.key", "pkey_pwd": "my_key_password" } Profile fields Mandatory: security_type, auth_type, ssid, identity, ca_path, cert_path, pkey_path Optional : bssid, pkey_pwd, dh_path, server_hostname

- EAP-PEAP/MSCHAPv2 Enterprise security:

{ "security_type": "enterprise", "auth_type": "eap_peap", "ssid": "my_eap_peap-mschapv2_ssid", "identity": "my_eap_peap_identity", "ca_path": "/data/ca.pem", "identity2": "eap-peap", "password": "password", "phase2auth":"MSCHAPv2" }

Profile fields

  - Mandatory: security_type, auth_type, ssid, identity, ca_path, identity2, password, phase2auth

  - Optional : bssid, server_hostname.

Note: For "phase2auth", the key MUST always be "phase2auth": while the VALUE can be, For example: "MSCHAPv2", "mschapv2" or "MsChAPv2" or any other similar lower/upper case letter combination.

Prerequisites:

To upload network profile into Talaria TWO file system, running of Gordon on the device is necessary.

- Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

- Write network profile JSON to Talaria TWO

Command syntax:

./script/storage.py [device] write <user_specify_path>/<user_spefcify_name>.json /data/<user_specify_name>.json

Note: It is a single line command.

Steps:

- Create a Network Profile File(JSON file).

Create a file and name anywhere in your PC and name it nprofile.json or <file_name>.json, and copy the following contents (we assume we are connecting to a WPA2-PSK network):

{ "security_type":"wpa23_psk", "ssid": "My-AP-Name", "passphrase": "0CD7AC607B" } nprofile.json file:

- To upload our network profile into Talaria file system, start gordon.elf and reset in bootloader mode.

./script/boot.py --reset=evk42_bl apps/gordon.elf

Figure 33: Starting gordon.elf

- Use storage.py to write the network config file (nprofile.json) into Talaria TWO file system in the partition /data.

./script/storage.py write nprofile.json /data/nprofile.json

Command Log:

Figure 34: Writing network config file

- Program (RAM/FLASH) application to connect to the Access Point configured in nprofile.json as:

  - Program RAM:

./script/boot.py --reset=evk42_bl examples/using_wifi/bin/wifi_connect.elf np_conf_path=/data/nprofile.json

Command Log:

Figure 35: Program RAM to connect to AP

  - Program FLASH:

python ./script/boot.py --reset=evk42_bl --flash=all examples/using_wifi/bin/wifi_connect.elf np_conf_path=/data/nprofile.json

Command Log:

Figure 36: Program Flash to connect to AP

## Windows Platform – libusbK Installed

### Reset Device

- Device Reset

Reset the device.

Command syntax:

python script\boot.py [device] --reset=evk42

Figure 37: Device reset

- Device Reset with Inhibiting-flash-boot

Reset the device with the inhibiting-flash-boot feature.

If the device has app code stored in flash from which the device will boot, this special reset command is required which will allow to flash a new app code.

Command syntax:

python script\boot.py [device] --reset=evk42_bl

Figure 38: Device Reset with Inhibiting-flash-boot

The device has been put into the programming mode (inhibiting-flash-boot).

- Reset with Inhibiting-flash-boot and booting Gordon

Reset the device with inhibiting-flash-boot and boot Gordon app.

This combination of commands is for the host to obtain access to the storage on Talaria TWO and operate read/write data.

Command syntax:

python script\boot.py [device] --reset=evk42_bl apps\gordon.elf

Figure 39: Reset with Inhibiting-flash-boot with booting Gordon

### Program App in Memory

Program app in Talaria TWO memory, assuming Talaria TWO has no code stored in flash to boot from.

Command syntax:

python script\boot.py [device] --reset=evk42 <path>\app.elf [boot_args_list]

Figure 40: Program app in Talaria TWO memory

### Erase Image or Program New Image in Flash

Erase Image in Flash

The following command erases the app code in flash:

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done so)

  - Flash empty.img

Command syntax:

python script\flash.py [device] write 0x1000 <path>\empty.img

Figure 41: Erase image in flash

Program New Image in Flash

Command syntax:

python script\boot.py [device] --reset=evk42_bl --flash=all <path>\app.elf [boot_args_list]

Figure 42: Program new image in flash

### Storage Data & File: Read/Write

- Read Partition Table from Device (to_json)

Reading file from device involves storage access. Hence, the running of Gordon on the device is necessary.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the flash.py with the command: to_json

Command syntax:

python script\flash.py [device] to_json <user_specify_path>\<user_spefcify_name>.json

Figure 43: Read Partition Table from device

Display of the board_0183.json file

Figure 44: Display of the board_0183.json file

- Write Partition Table in Device (from_json)

Writing file to device involves storage access. Hence, running of Gordon on the device is necessary.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the flash.py with the command: from_json

Command syntax:

python script\flash.py [device] from_json <user_specify_path>/<user_spefcify_name>.json

Figure 45: Write Partition Table in device

- Show File System Contents

Display the file system contents.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the storage.py with the command: tree

Command syntax:

python script\storage.py [device] tree

Figure 46: Show file system contents

- Read File and Store to Location on Host

Read file from the file system and store it in host.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the storage.py with the command: read

Command syntax:

python script\storage.py [device] read <path_read>\<file_name_read> --output <path_write>/<file_name_write>

Note: Single line command.

Figure 47: Read file and store to location on host

The file is created as per user entry:

Figure 48: File creation

- Write File to Device

Write file from host to the device in file system.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the storage.py with the command: write

Command syntax:

python script\storage.py [device] write <path_read>\<file_name_read> <path_write>\file_name_write>

Note: A single line command.

Figure 49: Write file to device

Use the ./script/storage.py tree command to check the new file on Talaria TWO file system:

Figure 50: Check new file on Talaria TWO file system

## Windows Platform – With COM Port

To retrieve the COM ports on the device, the libusbK drivers need to be uninstalled.

For more details on updating the libusbK drivers to COM port, refer section: Update Driver from libusbK to COM Port of document UG_Download_Tool.pdf (freertos_sdk_x.y\pc_tools\Download_Tool\doc).

Note: x and y in freertos_sdk_x.y refer to the SDK release version.

### Reset Device

All command reset is not supported under this setup.

Depending on the requirement to reset the device manually, execute the following steps:

- Reset the device manually:

  - Press the RESET button on Talaria TWO EVB

- Inhibiting-flash-boot (to re-enable programming):

  - Short GPIO17 to GND

  - Press the RESET button on EVB

  - Un-short GPIO17 and GND

  - Load gordon.elf from host, using boot.py

### Load Gordon

Load Gordon app to run the flash.py from host, assuming the image in device flash has been erased or has been placed in the inhibiting-flash-boot state.

Command syntax:

python script\boot.py [device] apps\gordon.elf

### Erase Image

The following command erases the app code in flash.

- Reset device with inhibiting-flash-boot and load gordon.elf (if not done so)

- Flash empty.img

Command syntax:

python script\flash.py [device] write 0x1000 <path>\empty.img

Figure 51: Erase image

Button reset to confirm the erase operation is a success.

### Storage Data & File: Read/Write

- Read Partition Table from Device (to_json)

Reading file from device involves storage access. Hence, running of Gordon on the device is necessary.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the flash.py with the command: to_json

Command syntax:

python script\flash.py [device] to_json <user_specify_path>\<user_spefcify_name>.json

Display of the board_0407.json file

Figure 52: Display of the board_0407.json file

- Write Partition Table in Device (from_json)

Writing file to device involves storage access. Hence, running of Gordon on the device is necessary.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the flash.py with the command: from_json

Command syntax:

python script\flash.py [device] from_json <user_specify_path>/<user_spefcify_name>.json

- Show File System Contents

Display the file system contents.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the storage.py with the command: tree

Command syntax:

python script\storage.py [device] tree

Figure 53: Show file system contents

- Read File and Store to Location on Host

Read file from the file system and store it in host.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the storage.py with the command: read

Command syntax:

python script\storage.py [device] read <path_read>\<file_name_read> --output <path_write>/<file_name_write>

Note: A single line command.

Figure 54: Read file and store to location on host

The file is created per user entry:

Figure 55: File creation

- Write File to Device

Write file from the host to the device in file system.

  - Reset device with inhibiting-flash-boot and load gordon.elf (if not done already)

  - Run the storage.py with the command: write

Command syntax:

python script\storage.py [device] write <path_read>\<file_name_read> <path_write>\file_name_write>

Note: A single line command.

Figure 56: Write file to device

Use the ./script/storage.py tree command to check the new file on Talaria TWO file system:

Figure 57: Check new file on Talaria TWO filesystem

### Sequence of CLI Commands

- Reset with inhibiting-flash-boot and load Gordon

Command Syntax:

./script/boot.py --device /dev/ttyUSB2 --reset=evk42_bl apps/gordon.elf

- Write the Partition Table

Write the Partition Table which is a standard configuration, standard_part_table.json, to Talaria TWO device:

Command Syntax:

./script/flash.py --device /dev/ttyUSB2 from_json tools/partition_files/standard_part_table.json

Note: A single line command.

- Flash New App to Device

Write a new application, iperf3 with bootargs, to the device.

Command Syntax:

./script/boot.py --device /dev/ttyUSB2 --reset=evk42_bl --flash=all ./bins/iperf3.elf ssid=TP-LINK_Guest_864F passphrase=test13579

Re-launch the console terminal app and reset Talaria TWO device.

## Flash New App to Device with an Existing App

Write the new app, stw.elf, to flash.

Command Syntax:

./script/boot.py --device /dev/ttyUSB2 --reset=evk42_bl --flash=all ./bins/stw.elf

Re-launch the console terminal app and reset Talaria TWO device.