- Create an AWS folder and clone the ADK from the cloud.

Git clone: https://github.com/aws-samples/amazon-kinesis-video-streams-media-interface.git

Figure 30: Clone ADK from the cloud

- Run the following command for MIPS GCC5.40

export PATH/home/xxxx/ingenic/tools/mips-gcc540-glibc222-64bit-r3.3.0/bin/:$PATH export CC=mips-linux-gnu-gcc export CXX=mips-linux-gnu-g++

Note: These exports can be added in the environment variables. Ensure to add the path for cross-compiler binaries.

Figure 31: Export - MIPS GCC5.40

- Copy dependent files from Ingenic SDK to Amazon ADK

cp -rf /home/xxx/INP3201_AWS_DEMO/T31_SNIPE_XXXXXXXX/FREERTOS_SDK_XXXXXXXX/ISVP-T31-1.1.5-XXXXXXXX/software/sdk/Ingenic-SDK-T31-1.1.5-XXXXXXXX/freertos_sdk/5.4.0/* /home/xxxx/INPIOT_GIT/AWS_KINESIS/amazon-kinesis-video-streams-media-interface/3rdparty/T31/

Figure 32: Copy dependent files

- Enter path: amazon-kinesis-video-streams-media-interface. Create a folder ‘build’ and enter the folder.

cd amazon-kinesis-video-streams-media-interface mkdir build; cd build;

Figure 33: Create build folder

- Check the cmake version installed. Run cmake to generate - makefile.

/home/xxxx/ingenic/tools/cmake-3.13.3/build/bin/cmake –version /home/xxxx/ingenic/tools/cmake-3.13.3/build/bin/cmake .. -DBUILD_WEBRTC_SAMPLES=ON -DBUILD_KVS_SAMPLES=ON -DBOARD=T31

Note: After configuring the environment, install cmake 3.13.3 to user path and not the system path.

Figure 34: Generate makefile

- Execute make to generate the executed file.

make -j4

Figure 35: Execute make

- Copy kvswebrtcmaster-static to the T31z filesystem, insmod the necessary drivers in T31z and execute: kvswebrtcmaster-static to stream the video to AWS cloud.