# Programming Talaria TWO over JTAG

The INP3000 Programmer Board version 3.0 has the JTAG interface to program Talaria TWO modules.

Install the appropriate dependencies for programming over JTAG.

## In Windows

- Open command prompt and reset the device in boot loader mode by executing the following command from the SDK directory:

.\script\reset.py evk42_bl

Figure 36: JTAG - Resetting the device in boot loader mode- console output (Windows)

- Open the command prompt and start OpenOCD by executing the following command from the SDK directory:

openocd -s .\conf -f ftdi.cfg -f t2.cfg

The following output is seen on command prompt console:

Figure 37: JTAG - Starting OpenOCD – console output

- Flash the default partition table by executing the following command:

.\pc_tools\T2_Flasher\bin\T2_Flasher_Windows.exe --operation=write_ptable .\<path to the standard_part_table.json file available in sdk_x.y/tools/partition_files directory>

Following output is observed after flashing the default partition table:

Figure 38: JTAG - Flashing the default partition table – console output

- In the same terminal, execute the following command from the SDK directory to flash the data image (data.img).

Note: This step is needed only if the application uses any certificates.

.\pc_tools\T2_Flasher\bin\T2_Flasher_Windows.exe --operation=write_part --partition=DATA .\<path to the generated data image>\data.img

The following output is observed after flashing the data image:

Figure 39: JTAG - Flashing the data image - console output

- In a separate command prompt window, execute the following command from the SDK directory to flash the application image:

.\pc_tools\T2_Flasher\bin\T2_Flasher_Windows.exe --operation=write_part --partition=BOOT .\<path to the generated application image>\app.img

The following output is observed after flashing the application image:

Figure 40: JTAG - Flashing the application image - console output

- In the same terminal, flash the VM image of the application by executing the following command:

.\pc_tools\T2_Flasher\bin\T2_Flasher_Windows.exe --operation=write_part --partition=VIRT .\<path to the generated application image.vm>

Following output is observed after flashing the application’s VM image:

Figure 41: JTAG - Flashing application's VM image – console output

The application is successfully flashed over JTAG. Now, OpenOCD needs to be terminated before debugging using Eclipse. Close all the command prompt windows to terminate OpenOCD.

## In Linux

- Open command prompt and reset the device in boot loader mode by executing the following command from the SDK directory:

./script/reset.py evk42_bl

Figure 42: JTAG - Resetting the device in boot loader mode- console output (Windows)

- Open command prompt and start OpenOCD by executing the following command from the SDK directory:

openocd -s ./conf -f ftdi.cfg -f t2.cfg

The following output is seen on command prompt console:

Figure 43: JTAG - Starting OpenOCD – console output

- Flash the default partition table by executing the following command:

.\pc_tools\T2_Flasher\bin\T2_Flasher_Linux --operation=write_ptable .\<path to the standard_part_table.json file available in sdk_x.y/tools/partition_files directory>

Following output is observed after flashing the default partition table:

Figure 44: JTAG - Flashing the default partition table – console output

- In the same terminal, execute the following command from the SDK directory to flash the data image (data.img).

Note: This step is needed only if the application uses any certificates.

.\pc_tools\T2_Flasher\bin\T2_Flasher_Linux --operation=write_part --partition=DATA .\<path to the generated data image>\data.img

The following output is observed after flashing the data image:

Figure 45: JTAG - Flashing the data image - console output

- In a separate command prompt window, execute the following command from the SDK directory to flash the application image:

.\pc_tools\T2_Flasher\bin\T2_Flasher_Linux --operation=write_part --partition=BOOT .\<path to the generated application image>\app.img

The following output is observed after flashing the application image:

Figure 46: JTAG - Flashing the application image - console output

- In the same terminal, flash the VM image of the application by executing the following command:

.\pc_tools\T2_Flasher\bin\T2_Flasher_Linux –operation=write_part –partition=VIRT .\<path to the generated application image.vm>\app.img.vm

Following output is observed after flashing the application’s VM image:

Figure 47: JTAG - Flashing application's VM image – console output

The application is successfully flashed over JTAG. Now, OpenOCD needs to be terminated before debugging using Eclipse. Close all the command prompt windows to terminate OpenOCD.