## BLE Provisioning

### Description

This use case is to demonstrate provisioning feature using BLE interface and configuring Talaria TWO as BLE peripheral mode. Start GATT server. Configure services, add characteristics, and advertise services. Connect BLE central and send SSID and Passphrase of your network using write operation from central (device with BLE app)

Talaria TWO receives credentials (SSID, Passphrase) and MCU triggers Wi-Fi connection manager to connect to the network using the received credentials.

### Prerequisites

Android device installed with native BLE app (BLE Scanner/nRF Connect).

### AT Command Sequence

| AT Command | Description |
| --- | --- |
| at | Checks the connection state |
| at+btinit | Initializes the BLE stack |
| at+blecfg=02:03:04:1f:cc:9e,0, InnoPhase_prov | Configures BLE device with parameters, BLE mac address: 02:03:04:1f:cc:9e BLE address type: 0 (BLE public address) BLE device name: InnoPhase_prov |
| at+blesrvstart | Starts BLE GATT Server |
| at+bleservcfg=11111111111111111111111111111111 | Creates unique custom service ID (32-bit long value) UUID: 11111111111111111111111111111111 |
| at+blechradd=11111111111111111111111111111111, 2A29,a,3 | Creates a unique custom characteristic ID for the above created custom service ID with parameters, Services uuid: 11111111111111111111111111111111 Char uuid: 2A29 Properties: "a" sets read and write property Permission: 3 sets read and write permission |
| at+bleservadd=11111111111111111111111111111111 | This includes the customized service into gatt server where, Custom service UUID: 11111111111111111111111111111111 |
| at+bleadvcfg=100,0,1600,0 | This configures the advertisement parameters where, Fast adv interval: 100 which equals 62.5ms (100 * 625 μs) Fast adv duration: 0 (continues advertisement) Slow adv interval: 1600, which equals 1000ms (1600 * 625 μs) Slow adv duration: 0 |
| at+bleadvstart | Start the BLE Advertisement |
| at+blecharwrdata=2a29,14 | Acknowledges BLE Characteristic Write Request with parameters, characteristic UUID: 2a29, data len (in hex format): 14 |
| at+wcon= IP,Inno@9070 | Connect to a desired AP from the received scan results Connect to WLAN Network as a Station with SSID= IP and Passphrase= Inno@9070 |
| at+wstatus=0 | Get IP address of Talaria TWO to verify the connection is successful |

Table 26: BLE Provisioning - AT Commands

### Procedure

Step 1: Execute the following commands on Talaria TWO:

at at+btinit at+blecfg=02:03:04:1f:cc:9e,0,InnoPhase_prov at+blesrvstart at+bleservcfg=11111111111111111111111111111111 at+blechradd=11111111111111111111111111111111,2A29,a,3 at+bleservadd=11111111111111111111111111111111 at+bleadvcfg=100,0,1600,0 at+bleadvstart

Figure 94: BLE Provisioning- serial log

Step 2: Launch BLE Scanner app from the Android device and connect to Talaria TWO GATT Server with the BLE name: InnoPhase_Prov.

Figure 95: InnoPhase_Prov-GATT server

Step 3: Once the BLE connection is established successfully, send data/Write Request by pressing W from BLE central. Here, add the SSID and passphrase of the network.

Figure 96: BLE write to add the SSID and Passphrase

Data received from the BLE central app on Talaria TWO. Send an acknowledgment of the write request from Talaria TWO using following command on serial terminal:

at+blecharwrdata=2a29,14

Figure 97: Acknowledgment of Write request

Step 4: Connect to the network with SSID=IP and Passphrase=Inno@9070.

at+wcon=IP,Inno@9070 at+wstatus=0

Figure 98: WLAN connection

## Secure BLE Provisioning Passkey method

### Description

This use case demonstrates secure BLE provisioning Passkey Entry STK generation method using mobile device as central.

Talaria TWO is configured as BLE peripheral which consists of configuring services, characteristics, SMP and advertise services. Initiate BLE connection from BLE central (device with BLE app) by entering the 6-digit passkey and send SSID and Passphrase of your network using write operation from central.

Talaria TWO receives credentials (SSID, Passphrase) and MCU triggers Wi-Fi connection manager to connect to the network using the received credentials.

### Prerequisites

Android/ iOS device installed with BLE app (BLE Scanner/nRF Connect).

### AT Command Sequence

| AT Command | Description |
| --- | --- |
| at | Checks the connection state |
| at+btinit | Initializes the BLE stack |
| at+blecfg=01:01:06:06:01:01,0,innophase | Configures BLE device with parameters, BLE mac address: 01:01:06:06:01:01 BLE address type: 0 (BLE public address) BLE device name: innophase |
| at+blesrvstart | Starts BLE GATT Server |
| at+bleservcfg=11111111111111111111111111111111 | Creates unique custom service ID (32-bit long value) UUID: 11111111111111111111111111111111 |
| at+blechradd=11111111111111111111111111111111, 0x2a19,a,3 | Creates a unique custom characteristic ID for the above created custom service ID with parameters, Services uuid: 11111111111111111111111111111111 Char uuid: 0x2a19 Properties: "a" sets read and write property Permission: 3 sets read and write permission |
| at+bleservadd=11111111111111111111111111111111 | This includes the customized service into gatt server where, Custom service UUID: 11111111111111111111111111111111 |
| at+bleadvcfg=32,0,160,480 | This configures the advertisement parameters where, Fast adv interval: 32 which equals 20ms (32 * 625 μs) Fast adv duration: 0 (continues advertisement) Slow adv interval: 160, which equals 100ms (160 * 625 μs) Slow adv duration: 480 which equals 300ms (480 * 625 μs) |
| at+blesmpcfg=0,0,1,1,0,0,16,1 | Configures the SMP (security) with parameters, io cap: 0 (display_only) oob : 0 (default) bondable: 1 mitm: 1 sc: 0 keypress: 0 key size min: 16 bytes encrypt: 1 |
| at+bleadvstart | Start BLE advertisement |
| at+blecharwrdata=2a19,1F | Acknowledges BLE Characteristic Write Request with parameters, characteristic UUID: 2a19, data len(in hex format): 1F |
| at+wcon= InnoPhase_AE,Inno@1234 | Connect to WLAN Network as a Station with SSID= InnoPhase_AE and Passphrase= Inno@1234 |

Table 27: BLE Provisioning - AT Commands

For more details on the AT command parameters, refer BT/BLE commands section of UG_AT_Commands.pdf (freertos_sdk_x.y\binaries\product\at\doc).

### Procedure

Step 1: Execute the following commands on Talaria TWO:

at at+btinit at+blecfg=01:01:06:06:01:01,0,innophase at+blesrvstart at+bleservcfg=11111111111111111111111111111111 at+blechradd=11111111111111111111111111111111, 0x2a19,a,3 at+bleservadd=11111111111111111111111111111111 at+bleadvcfg=32,0,160,480 at+blesmpcfg=0,0,1,1,0,0,16,1 at+bleadvstart

Figure 99: BLE Provisioning- serial log

Step 2: Launch nRF Connect app from the iOS device and scan. Talaria TWO GATT Server with the BLE name innophase will be listed. Click on connect.

Figure 100: innophase-GATT server

The passkey for pairing will be generated and shown on the AT command line.

Figure 101: Randomly generated 6-digit Passkey

Step 3: Add the 6-digit passkey “707476” to the app and click on Pair. Now the BLE GATT connection is created.

Figure 102: Pairing request

Step 4: Once the BLE connection is established successfully, send data/Write Request by pressing W from BLE central. Here, add the SSID and passphrase of the network.

Figure 103: BLE write to add the SSID and Passphrase

An asynchronous message on Talaria TWO will be displayed for the Write Request sent from the BLE central app.

Figure 104: Async data reception from mobile app

Send an acknowledgment of the write request from Talaria TWO using following command:

at+blecharwrdata=2a19,1F

Figure 105: Acknowledgment of Write request

Step 4: Connect to the network with SSID= InnoPhase_AE and Passphrase= Inno@1234.

at+wcon= InnoPhase_AE,Inno@1234

Figure 106: WLAN connection

## Secure BLE Provisioning Just Works Method

### Description

This use case is to demonstrate secure BLE provisioning Just Works STK generation method using two Talaria TWO EVBs (say T1 and T2), one as peripheral and other as central.

One Talaria TWO EVB (T1) is configured as BLE peripheral which consists of configuring services, characteristics, SMP and advertise services. Initiate BLE connection from BLE central (another Talaria TWO module) and send SSID and Passphrase of your network.

T1 receives credentials (SSID, Passphrase) and MCU triggers Wi-Fi connection manager to connect to the network using the received credentials.

### Prerequisites

Two Talaria TWO devices programmed with t2_atcmds.elf

### AT Command Sequence

| AT Command | Description |
| --- | --- |
| at | Checks the connection state |
| at+btinit | Initializes the BLE stack |
| at+blecfg=01:01:06:06:01:01,0,innophase | Configures BLE device with parameters, BLE mac address: 01:01:06:06:01:01 BLE address type: 0 (BLE public address) BLE device name: innophase |
| at+blesrvstart | Starts BLE GATT Server |
| at+bleservcfg=11111111111111111111111111111111 | Creates unique custom service ID (32-bit long value) UUID: 11111111111111111111111111111111 |
| at+blechradd=11111111111111111111111111111111, 0x2a19,a,3 | Creates a unique custom characteristic ID for the above created custom service ID with parameters, Services uuid: 11111111111111111111111111111111 Char uuid: 0x2a19 Properties: "a" sets read and write property Permission: 3 sets read and write permission |
| at+bleservadd=11111111111111111111111111111111 | This includes the customized service into gatt server where, Custom service UUID: 11111111111111111111111111111111 |
| at+bleadvcfg=32,0,160,480 | This configures the advertisement parameters where, Fast adv interval: 32 which equals 20ms (32 * 625 μs) Fast adv duration: 0 (continues advertisement) Slow adv interval: 160, which equals 100ms (160 * 625 μs) Slow adv duration: 480 which equals 300ms (480 * 625 μs) |
| at+blesmpcfg=0,0,1,1,0,0,16,1 | Configures the SMP (security) with parameters, io cap: 0 (display_only) oob : 0 (default) bondable: 1 mitm: 1 sc: 0 keypress: 0 key size min: 16 bytes encrypt: 1 |
| at+bleadvstart | Start BLE advertisement |
| at+blecharwrdata=2a19,1F | Acknowledges BLE Characteristic Write Request with parameters, characteristic UUID: 2a19, data len(in hex format): 1F |
| at+wcon= InnoPhase_AE,Inno@1234 | Connect to WLAN Network as a Station with SSID= InnoPhase_AE and Passphrase= Inno@1234 |

Table 28: BLE Provisioning - AT Commands

For more details on the AT command, refer BT/BLE commands section of UG_AT_Commands.pdf

### Procedure

Step 1: On the first Talaria TWO board (BLE peripheral), execute the following commands:

at at+btinit at+blecfg=02:01:04:1f:cc:9e,0,innophase at+blesrvstart at+bleservcfg=11111111111111111111111111111111 at+blechradd=11111111111111111111111111111111,0x2a19,a,3 at+bleservadd=11111111111111111111111111111111 at+bleadvcfg=32,0,160,480 at+blesmpcfg=0,0,1,1,0,0,16,1 at+bleadvstart

Figure 107: BLE Peripheral- serial log

Step 2: Configure the second Talaria TWO board as BLE central. Scan for BLE devices and connect to the desired BLE device.

Configure the SMP using at+bleauth with parameters connection id: 0, oob:0, bondable:1, Mitm: 0, sc:1 and keypress: 0.

Write GATT characteristic with parameters connection id: 0, handle: 1, length: 51 bytes, data: 31 and Send SSID and PWD for the desired network using at+blecharw command.

at at+btinit at+blescancfg=5000,96,48,96,24,1 at+blecfg=02:03:05:05:03:02,0,central at+blescan at+blecon=9e-cc-1f-04-01-02,0 at+bleauth=0,0,1,0,1,0 at+blecharwr=0,1,51,31,SSID=InnoPhase_AE:PWD=Inno@1234

Figure 108: BLE central- serial log

For the data received from the BLE central device, Send an acknowledgment of the write request from peripheral.

at+blecharwrdata=2a19,1f

Figure 109: Acknowledgement of Write request

Step 4: Issue at+wcon command to connect to an AP with SSID "InnoPhase_AE" and passphrase "Inno@1234" from the first EVB(T1)

at+wcon= InnoPhase_AE,Inno@1234

Figure 110: WLAN connection

## FOTA

Firmware-Over-the-Air (FOTA) allows wireless delivery of firmware updates or configurations to the embedded devices.

### Description

This use case describes the FOTA process for Talaria TWO EVB using the Talaria TWO AT commands with details on implementing and triggering FOTA feature

### Prerequisites

- Apache server/HFS or any HTTP server to store Talaria TWO ELF (t2_atcmds.elf) and FOTA configuration file (fota_config.json).

- Access Point configured with WPA/WPA2/WPA3 personal/enterprise security.

- Minicom Serial terminal for host interface to execute AT commands.

### AT Command Sequence

| AT Command | Description |
| --- | --- |
| at | To check the connection state |
| at+wcon | Connect to secured Access Point |
| at+wstatus | Get IP address of Talaria TWO to verify if the connection (L2+L3) is successful |
| at+fotacfgadd | This command is optional and is required to update the configuration file present in the root filesystem of Talaria TWO |
| at+fota | Start FOTA operation |

Table 29: FOTA - AT commands

### Procedure

The FOTA demo process involves the following steps:

- Generating hash value for atcmds.elf.

- Modify fota_config.json file.

- Server setup.

- Program and running the application:

  - Generate root filesystem and program filesystem, application image(t2_atcmds.elf) and partition table (ssbl_part_table.json)

  - Issue the AT commands to connect to network and trigger FOTA.

Generate the hash value:

Execute the following command on ubuntu terminal from SDK directory to generate the hash value:

sha256sum binaries/product/at/bin/atcmds.elf

Figure 111: Generate the hash value

Modification of fota_config.json file:

Change the following fields in the fota_config.json (freertos_sdk_x.y\binaries\product\at\fs\fota_config.json):

- “hostname” as IP address based on server IP address.

- “uri” with the path of the fota_config.json and t2_atcmds.elf stored at the Apache server.

- “hash” value.

{ "package_version" : "2.0", "files" : [ { "type" : "configuration", "name" : "fota.config", "protocol" : "http", "hostname" : "192.168.201.233", "port" : 80, "secured" : 0, "uri" : "/FOTA/fota_config.json" }, { "type" : "firmware", "name" : "atcmd", "version" : "2.1", "protocol" : "http", "hostname" : "192.168.201.233", "port" : 80, "secured" : 0, "uri" : "/FOTA/t2_atcmds.elf", "hash" : "a93ce95fddea0f635f726311d0f8c912909858bfb8c9b4d4204ec5971d9d1aa4 " } ] }

Server Setup:

- Using Apache:

  - Apache server is set up in Windows OS.

  - Download the Apache server from the following link: https://httpd.apache.org/docs/current/platform/windows.html#down

Figure 112: Apache server setup link

  - Place the modified fota_config.json and t2_atcmds.elf under the Apache folder htdocs by creating any default folder.

For example: FOTA. The same path is added in the “uri” section of fota_config.json file.

Figure 113: Folder to add the JSON and elf file

Note: If using hash to verify the image integrity, ensure to place the same t2_atcmds.elf in \Apache24\htdocs\FOTA folder for which hash was generated.

- Using HFS (HTTP File Server):

  - The HTTP File Server can be downloaded from the following link: https://github.com/rejetto/hfs/releases

Figure 114: HTTP File Server setup link

  - Open the HFS tool and create a folder (for example: FOTA) and to this, copy the  modified fota_config.json and t2_atcmds.elf. Same path is added in the “uri” section of fota_config.json file.

Figure 115: Folder to add the JSON and elf file

  - Check the server accessibility by sending HTTP GET from the browser as shown:

Open Browser-> Enter IP address followed by resource path i.e., "/192.168.1.216/FOTA“ where 192.168.1.216 is the server IP address (IP address of the laptop) and 80 is the server port.

Figure 116: HTTP get from browser to verify server accessibility

Programming and running the application:

- In the ubuntu terminal, run the following command from the SDK directory to generate root image and flash root image, SSBL partition table and t2_atcmds.elf.

python3 ./script/program_flash.py --no_reset -i binaries/product/at/bin/atcmds.elf -spt tools/partition_files/ssbl_part_table.json

Figure 117 Generated root image

Figure 118: Flashing the AT command ELF

- In any terminal window, open miniterm at baud rate of 2457600 to view CONSOLE logs for Talaria TWO.

miniterm.py /dev/ttyUSB3 2457600

Figure 119: Miniterm console

Note: After programming Talaria TWO, it is recommended to reset the module either using the following command from SDK directory or press the reset button on the EVB.

./script/boot.py --device /dev/ttyUSB2 --reset=evk42

Figure 120: Talaria TWO reset – console

- The module boots up with the programmed at command ELF. Issue the following commands on the serial terminal to perform the FOTA operation.

at at+wcon=InnoPhase,Inno@1234 at+wstatus=0 at+fota=1

Note: Updating the configuration file in Talaria TWO’s root filesystem:

  - Navigate to the config file (sdk_x.y\binaries\product\at\fs) and modify the fota_config.json file.

  - Execute the following command to update the FOTA configuration file in the root file system of Talaria TWO:

  - Open the miniterm (miniterm.py /dev/ttyUSB3 2457600) to check the console logs on Talaria TWO

at+fotacfgadd=<len>

For example: at+fotacfgadd=652

  - Transfer the updated config file from minicom.

Press CTRL-A Z-> Enter S -> Select ascii -> Select folder where config is located ->  select continue.

Figure 121: Updated file received - Talaria TWO console

### Serial Log

Figure 122: FOTA - serial log

Talaria TWO console output when at+fota command is issued on the serial terminal:

CMD:~Hexdump of uartbuf before processing, len=10 7E 04 00 00 00 10 0E 00 63 00 ¦ ~.......c. Zero arguments cmd:~:2 resp-len:28 CMD:atHexdump of uartbuf before processing, len=3 61 74 00 ¦ at. Zero arguments cmd:at:2 Ready resp-len:9 CMD:at+wcon=InnoPhase,Inno@1234Hexdump of uartbuf before processing, len=28 61 74 2B 77 63 6F 6E 3D 49 6E 6E 6F 50 68 61 73 ¦ at+wcon=InnoPhas 65 2C 49 6E 6E 6F 40 31 32 33 34 00 ¦ e,Inno@1234. cmd:at+wcon=InnoPhase,Inno@1234:8 arg:InnoPhase,Inno@1234 ssid:0x00059e2c passphrase:0x00059e36SSID:InnoPhase,timeo=0xaba9500 add_network_profile:0 [43.201,304] CONNECT:b0:95:75:5c:d2:8f Channel:11 rssi:-8 dBm MSG:202 notify MSG:200 notify[43.242,749] MYIP 192.168.0.146 [43.242,913] IPv6 [fe80::e269:3aff:fe00:167e]-link MSG:204 notify resp-len:9 CMD:at+wstatus=0Hexdump of uartbuf before processing, len=13 61 74 2B 77 73 74 61 74 75 73 3D 30 00 ¦ at+wstatus=0. cmd:at+wstatus=0:11 arg:0 str:0:1 string : 0 : endptr : :0 IP:192.168.0.146:255.255.255.0:192.168.0.1 resp-len:49 resp-len:9 CMD:at+fota=1Hexdump of uartbuf before processing, len=10 61 74 2B 66 6F 74 61 3D 31 00 ¦ at+fota=1. cmd:at+fota=1:8 arg:1 option:1: str:1:1 string : 1 : endptr : :0 End of command line option:<null>: fota_json_init: /root/fota_config.json f = 0x000be1f8 Parsing rootfs FOTA config file*** package_version = 1.0 Package version = 1.0 type = configuration name = fota.config version, <null> protocol = http hostname = 192.168.0.177 port = 80 secured = 0 uri = /FOTA/fota_config.json url, <null> hash, <null> configuration fota.config http 192.168.0.177 80 0 /FOTA/fota_config.json <null> <null> type = firmware name = atcmd version = 2.2 protocol = http hostname = 192.168.0.177 port = 80 secured = 0 uri = /FOTA/t2_atcmds.elf url, <null> hash = 34aadc96b19b4aff23b08400523d427435be8aa0ff82b6084820165ceadc9174 firmware atcmd http 192.168.0.177 80 0 /FOTA/t2_atcmds.elf <null> 2.2 Fota Init Success: bc388 Performing Fota..cc check_for_update = 1 auto_reset=1 fota_perform check_for_update = 1 fota_config_file_download 1049 fota_http_connect:host=192.168.0.177 port=80 Calling http_client_open() fota_config_file_download 1062 Parsing Remote FOTA config file*** package_version = 2.0 Package version = 2.0 type = configuration name = fota.config version, <null> protocol = http hostname = 192.168.0.177 port = 80 secured = 0 uri = /FOTA/fota_config.json url, <null> hash, <null> configuration fota.config http 192.168.0.177 80 0 /FOTA/fota_config.json <null> <null> type = firmware name = atcmd version = 2.1 protocol = http hostname = 192.168.0.177 port = 80 secured = 0 uri = /FOTA/t2_atcmds.elf url, <null> hash = 34aadc96b19b4aff23b08400523d427435be8aa0ff82b6084820165ceadc9174 firmware atcmd http 192.168.0.177 80 0 /FOTA/t2_atcmds.elf <null> 2.1 utils_num_str_cmp 2 0 1 0 deci1 = 2, fracn1 = 0, deci2 = 1, fracn2 = 0 Using the Remote config (Newly fetched) file type = configuration type = firmware fota_json_init: /root/part.json f = 0x000bd3c8 Image array size = 2 name = atcmd version = 1.0 start_sector = 32 1.0 :32 name = atcmd version = 0.0 start_sector = 232 0.0 :232 utils_num_str_cmp 1 0 0 0 deci1 = 1, fracn1 = 0, deci2 = 0, fracn2 = 0 Selected index = 1 Download the new f/w @ sector = 232 fota_http_connect:host=192.168.0.177 port=80 Calling http_client_open(). fota_http_cb: resp->resp_len = 1252, resp->resp_total_len = 614932 total_rcvd_len= 1252 fota_http_cb: resp->resp_len = 1460, resp->resp_total_len = 614932 total_rcvd_len= 541452 fota_http_cb: resp->resp_len = 1460, resp->resp_total_len = 614932 total_rcvd_len= 579412 fota_http_cb: resp->resp_len = 1460, resp->resp_total_len = 614932 total_rcvd_len= 614452 . fota_http_cb: resp->resp_len = 480, resp->resp_total_len = 614932 total_rcvd_len= 614932 sector_cache_flush_all Fw download complete image size = 614932 image hash: 34aadc96b19b4aff23b0840523d427435be8aa0ff82b684820165ceadc9174 hash from cfg: 34aadc96b19b4aff23b0840523d427435be8aa0ff82b684820165ceadc9174 Image integity verified. ok. next index = 1 Performing Fota Commit..... fota_commit utils_num_str_add 0 0 2 0 deci1 = 0, fracn1 = 0, deci2 = 2, fracn2 = 0 utils_num_str_add : out_str = 2.0 fota_update_part_file: !!!Updated new version = 2.0 fota_json_init: /root/boot.json f = 0x000b3718 Setting next boot index = 1 Y-BOOT 208ef13 2019-07-22 12:26:54 -0500 790da1-b-7 ROM yoda-h0-rom-16-0-gd5a8e586 FLASH:PWAE WWWWWAE[0.017,903] heapsize is less than requested 29952 < 30000 Build $Id: git-4304eac2 $Build $Id: git-4304eac2 $ Flash detected. flash.hw.uuid: 39483937-3207-00b0-0064-ffffffffffff Bootargs: vm.flash_location=0x0003fc00 passphrase=123467890 ssid=innotest Application Information: ------------------------ Name : atcmd Version : 2.0 Build Date : Oct 25 2023 Build Time : 23:34:21 Heap Available: 268 KB (274456 Bytes) $App:git-6f5f99b SDK Ver: FREERTOS_SDK_1.0 At Command App addr e0:69:3a:00:16:d4 domain:1-11@20before: magic1=0x0, val=0x0, magic2=0x0 Crash detection logic initialized after: magic1=0x11223344, val=0x0, magic2=0x55667788 Serial-to-Wireless: Ready starting thread-sock SSBL Application Boot indx = 1 Getting Addres of first virt section, name = .virt0 offset = 16c00 vm_flash_location= =fec00 Elf Load OK... Boot-args: vm.flash_location=0x000fec00 Build $Id: git-4304eac2 $ Flash detected. flash.hw.uuid: 39483937-3207-00b0-0064-ffffffffffff Bootargs: vm.flash_location=0x0003fc00 passphrase=123467890 ssid=innotest Application Information: ------------------------ Name : atcmd Version : 2.0 Build Date : Oct 25 2023 Build Time : 23:34:21 Heap Available: 268 KB (274456 Bytes) $App:git-6f5f99b SDK Ver: FREERTOS_SDK_1.0 At Command App addr e0:69:3a:00:16:d4 domain:1-11@20before: magic1=0x0, val=0x0, magic2=0x0 Crash detection logic initialized after: magic1=0x11223344, val=0x0, magic2=0x55667788 Serial-to-Wireless: Ready starting thread-sock