## Programming Talaria TWO

### Hardware Set-up

Figure 8: Hardware set-up

Note:

- For more details on the hardware requirements, refer section: Hardware Requirements of the document: UG_Dual-Stack.pdf (talaria_two_dual_stack_vx.y\host\INP3201\doc).

- For pin-outs details on INP3000 programmer board, refer UG_Programming_using_INP3000.pdf (freertos_sdk_x.y\doc\user_guides\ug_programming_using_INP3000).

### Procedure

Program the Dual-Stack ELF (<dual_stack_sdio>.elf) onto Talaria TWO using the following steps:

- Open a terminal on console machine and start openocd from FreeRTOS SDK root folder (freertos_sdk_x.y\). This enables in-system programming on Talaria TWO.

Command:

sudo openocd -s ./conf/ -f ftdi_swd.cfg -f t2_swd.cfg

Expected output:

Open On-Chip Debugger 0.10.0 Licensed under GNU GPL v2 For bug reports, read http://openocd.org/doc/doxygen/bugs.html adapter speed: 1000 kHz Info : FTDI SWD mode enabled cortex_m reset_config sysresetreq Info : clock speed 1000 kHz Info : SWD DPIDR 0x2ba01477 Info : talaria_two.cpu: hardware has 6 breakpoints, 4 watchpoints Info : Talaria TWO rev. H0 cortex_m reset_config sysresetreq Info : clock speed 1000 kHz Info : SWD DPIDR 0x2ba01477 Info : talaria_two.cpu: hardware has 6 breakpoints, 4 watchpoints Info : Talaria TWO rev. H0

- Start arden.py script from SDK root folder (sdk_x.y\) on a new terminal:

Command:

sudo python3 ./script/arden.py ./apps/gordon-jtag/bin/gordon-jtag.elf

Expected output:

[sudo] password for dell10: (re)connecting with openocd openocd version: 'Open On-Chip Debugger 0.10.0' boot rom version of connected device: "yoda-h0-rom-16-0-gd5a8e586" starting up on localhost port 10000

- Run the script gdbrun.py from FreeRTOS SDK root folder (freertos_sdk_x.y\) on a new terminal. Given a HOST with running OpenOCD and UART relay, gdbrun.py connects to Host and load/boot an ELF image.

Command:

sudo python3 ./script/gdbrun.py ./apps/gordon-jtag/bin/gordon-jtag.elf --noconsole --nowait

- The <dual_stack_sdio>.elf generates dualstack.img and dualstack.img.vm and they are flashed to Talaria TWO using the following sequence of commands.

Command:

sudo python3 ./script/boot.py --output dualstack.img ./solutions/dual_stack/bin/dual_stack_sdio.elf hio.transport=sdio hio.maxsize=8192 hio.sdio_mhz=10 wifi.outq_max=32 hio.irq_min_gap=60 hio.irq_retry_time=200 wifi.rts=2 wifi.pmode_cts=1

Note:

  - To enable Host reboot when Talaria TWO reboots, use the following bootargs:

krn.gpio=--------------p----p-p ds.host_pwroff_gpio_state=LLLLLL--------p--p--p-

  - For video streaming, use the following bootargs:

hio.min_heap_for_burst_tx=24000 hio.max_data_ind=3

- The minimum heap required for burst mechanism is 24000.

- hio.max_data_ind specifies the maximum number of packets which can be queued in HIO for transmission.

Expected output:

Loading ./solutions/dual_stack/bin/dual_stack_sdio.elf Entrypoint 0xb1159 Wrote bootimage to .img 103_972 bytes Wrote vmimage to dualstack.img.vm 482_176 bytes

Command:

./script/flash.py --device localhost:10000 part_write BOOT @root/ freertos_sdk_x.y/dualstack.img

Note: Replace x and y in freertos_sdk_x.y with the appropriate SDK release version.

Expected output:

write: [==================================================================>] 103972 bytes written to addr 4096

Command:

./script/flash.py --device localhost:10000 part_write VIRT @root/ freertos_sdk_x.y/dualstack.img.vm

Expected output:

write: [==================================================================>] 482176 bytes written to addr 262144

After flashing the dual_stack_sdio.elf, update the root.img with AWS certificates on /data partition.

Copy the AWS IoT certificates (aws_device_cert.crt, aws_device_pkey.key & aws_root_ca.crt) into the rootfs folder (freertos_sdk_x.y/root_fs/root) in the SDK package and use mklittlefs command to create a new root image.

./tools/mklittlefs/mklittlefs -s 0x40000 -c ./root_fs/root root.img

Command:

./pc_tools/T2_Flasher/bin/T2_Flasher_Linux --operation=write_part --partition=DATA ./root.img

Expected output:

/tmp/_MEItbY8QH/bin Starting OpenOCD /tmp/_MEItbY8QH/bin/openocd/linux64/openocd_linux64 -s /tmp/_MEItbY8QH/bin/sdk-files -f /tmp/_MEItbY8QH/bin/openocd/interface/cmsis-dap.cfg -f t2_swd.cfg xPack OpenOCD, x86_64 Open On-Chip Debugger 0.10.0+dev (2020-10-13-17:27) Licensed under GNU GPL v2 For bug reports, read http://openocd.org/doc/doxygen/bugs.html DEPRECATED! use 'adapter speed' not 'adapter_khz' cortex_m reset_config sysresetreq Error: couldn't bind tcl to socket on port 6666: Address already in use (re)connecting with openocd Info : accepting 'tcl' connection on tcp/6666 talaria_two Open On-Chip Debugger 0.10.0 openocd version: 'Open On-Chip Debugger 0.10.0' Connecting GDB Info : accepting 'gdb' connection on tcp/3333 GDB: monitor reset halt Info : Talaria TWO rev. H0 target halted due to debug-request, current mode: Thread xPSR: 0x01000000 pc: 0x00020f90 msp: 0x00041a78 GDB: gdb load /tmp/_MEItbY8QH/bin/tinyflasher.elf GDB: write WAIT_OP at 0x7ffe8 Starting TinyFlasher GDB: gdb continue GDB: write READ_PTABLE_OP at 0x7ffe8 GDB: gdb continue waiting... operation complete GDB: write WAIT_OP at 0x7ffe8 Read ptable contents from memory Flashing ./root.img at 0x100000 GDB: write sector count at 0x7fff0 GDB: write target flash addr at 0x7fff4 GDB: write image binary to RAM staging location (0x80000) GDB: write FLASH_OP at 0x7ffe8 GDB: gdb continue waiting... operation complete GDB: write WAIT_OP at 0x7ffe8 Disconnecting OpenOCD DONE Info : dropped 'tcl' connection Info : dropped 'gdb' connection

Once flashing and root image update is complete, reset Talaria TWO and reboot T31ZX to start the demo application.