https://www.st.com/en/development-tools/stm32cubeprog.html

Execute the following steps to flash the binary file using STM32Cube programmer:

Figure 1: MCU ST-Link programmer connection

- On launching the STM32Cube programmer, the following window pops-up.

Figure 2: STM32Cube programmer board

- To load the binary file, click on ‘Open File’ tab in the main window, select the .elf or .bin file to program from the Open File pop-up window and click Open.

Figure 3: Load the binary file

The selected .elf or .bin file will be loaded in the STM32Cube programmer. ST-Link configuration can be seen on the right-side.

- Port: SWD

- Frequency: 4000 Khz

- Mode: Normal

- Access Port: 0

- Reset mode: Software reset

- Speed: Reliable

- Shared: Disabled

- On confirming the ST-Link configurations, click on CONNECT button on the top right corner to connect to the target bard.

Figure 4: Target window – device connected

- Click on the Download button to program the STM32 MCU:

Figure 5: Target -> Program & Verify

Once the programming is complete, a pop-up window with the following message is displayed: File download completed. Click on OK to continue and close the STM32Cube programming software.

- After flashing the binary onto the MCU, press the reset button on the INP3201 board. This will turn the T31ZX Host ON and execute the app_init.sh script automatically.

The console output confirms the binary being loaded appropriately.