This application note demonstrates the use of APIs to create a provisioning application using BLE as the mode of transferring provisioning data. A reusable module is provided for provisioning the device in the following location: freertos_sdk_x.y\components\prov. The code in this sample application can be used in any user application to use the provisioning feature.

For more information regarding the module and the APIs, refer: prov_apiref.pdf.

# Topology

Talaria TWO

BLE Mobile Application

Access Point

BLE

Wi-Fi

Provisioning

Talaria TWO

BLE Mobile Application

Access Point

BLE

Wi-Fi

Provisioning

Figure 1: Topology

# List of APIs

- prov_start(): Initializes and starts the Provisioning service.

- prov_conf_param_str_get(): Gets the configuration parameters of type string.

- prov_conf_param_int_get(): Gets the configuration parameter of type integer.

- prov_is_provisioned(): Checks if the device is provisioned at least once.

- wcm_create(): Creates a Wi-Fi network interface.

- wcm_add_network_profile(): Adds a network profile to WCM.

- wcm_auto_connect(): Starts or stops auto connect.

- wcm_notify_enable(): Enables callbacks of link and IP address changes.

# Code Walkthrough

## Application Flow

In this application, provisioning is done over BLE, and the configuration data is stored in the file system.

Following are the steps:

- Initiate BLE provisioning by creating a GATT server.

- Provision the SSID and passphrase by selecting the network from the SSID list created from scan results using the BLE Provisioning mobile app.

- Store the configuration data.

- Check Wi-Fi connectivity using the configured SSID and passphrase.

## Sample Code Walkthrough

A user-defined data struct is created to store provisioning data.

struct param_t { const char *ssid; const char *passphrase; const char *dummy; };

prov_start_prams_t and prov_cfg is the pointer to the data structure of type prov_start_prams_t.

Provisioning method, device name, appearance, manufacturer name, user callback and context pointer are passed to structure members of prov_start_prams_t.

if(0 == prov_is_provisioned()){ /*Provisioning*/ prov_start_prams_t prov_cfg = {0}; prov_cfg.method = PROV_METHOD_BLE; prov_cfg.cb = app_prov_cb; /*"custparam" is configurable parameter present in part.json file provisioning module does not add this parametr bu default to the prov service. Need to add it exlicietly*/ prov_custom_param_add(0xAA44, "custparam"); prov_start(&prov_cfg); os_sem_wait(&app_lock); }

prov_start API starts the provisioning service and prov_is_provisioned checks if provisioning is complete. Here, the prov_conf_param_str_get API is used to get the provisioning parameters such as SSID and passphrase. These parameters are stored in user defined structure param_t.

/*if provisioning is done. use provisioned params*/ if(prov_is_provisioned()){ param.ssid = prov_conf_param_str_get("ssid"); param.passphrase = prov_conf_param_str_get("passphrase"); os_printf("\n[APP] %s %s", param.ssid, param.passphrase); }

wcm_create()API creates a Wi-Fi network interface and connects to network using provisioned parameters.

wcm_handle = wcm_create(NULL); app_wcm_connect(wcm_handle, (char *)param.ssid, (char *)param.passphrase); if(!wcm_connect_success) { os_printf("\n[APP]Error: Failed to connect to WiFi N/w"); goto exit; } os_printf("\n[APP]Success: Connected to %s N/w", (char *)param.ssid);

# Running the Application

## Write the Provisioning File into Talaria TWO Filesystem

Using the Download Tool’s Write Files feature, write the file (part.json) present in freertos_sdk_x.y\examples\prov\data into Talaria TWO filesystem.

This can be accomplished by using the option: Filesystem->Write Files from a directory->(File Open Icon). Choose the directory sdk_x.y\examples\prov\data and click on Write Files option.

Note: To re-provision the application, write the files using the procedure described above.

## Programming Talaria TWO board with ELF

Program prov_app.elf (examples\prov\bin\prov_app.elf) using the Download tool.

- Launch the Download tool provided with InnoPhase Talaria TWO SDK.

- In the GUI window:

  - Boot Target: Select the appropriate EVK from the drop-down.

  - ELF Input: Load the prov_app.elf by clicking on Select ELF File.

  - Programming: Click on Prog Flash.

Once done, follow the instructions mentioned in section 7.3 to run the mobile

Note:

- There is no requirement of boot arguments. User can run the application directly. On running the application, it creates a provisioning GATT server.

## Running the Application using Android or iOS App

- Download and install the Android or iOS mobile application from the play/app store using the following links:

  - Android:https://play.google.com/store/apps/details?id=com.innophase.provisioning&hl=en&gl=US

  - iOS: https://apps.apple.com/in/app/ble-provisioning/id1627682345

Figure 2: Android mobile app in play store

Figure 3: iOS mobile app in app store

- BLE Provisioning can be enabled via non-secure and secure BLE connections. Following are the three supported methods:

  - Non-secure provisioning

  - Secure Provisioning – Just Work Method

  - Secure Provisioning – Passkey Method

- Non-secure Provisioning

On flashing prov_app.elf onto Talaria TWO using the Download Tool, the following console logs are observed.

Talaria TWO console output:

Y-BOOT 208ef13 2019-07-22 12:26:54 -0500 790da1-b-7 ROM yoda-h0-rom-16-0-gd5a8e586 FLASH:PWWWWWWAE Build $Id: git-2eebd42 $ Flash detected. flash.hw.uuid: 39483937-3207-0080-0055-ffffffffffff Bootargs: prov_every_time=1 Application Information: ------------------------ Name : Prov App Version : 1.0 Build Date : Sep 8 2023 Build Time : 13:58:53 Heap Available: 297 KB (304152 Bytes) [APP]Bootparams : -------------------- ssid= <null> passphrase= <null> dummy= <null> secured_prov =0 pass_key=0 prov_every_time=1 [APP]Bootparams end here.... [PROV]prov_is_provisioned : val = 1 [PROV] bt_gap_init [PROV] common_server_create: Inno_Provisioning InnophaseInc 0 [PROV] bt_gatt_create_service_128 ret = 0x000bf278 [PROV] Adding Characteristics [PROV] bt_gatt_add_service [PROV] bt_gap_discoverable_mode, ret = 0

- Secure Provisioning

In case of secure BLE provisioning, flash the prov_app.elf onto Talaria TWO with the following boot arguments as applicable:

  - Just Work Method: secured_prov=1,pass_key=0, or

  - Passkey Method: secured_prov=1,pass_key=1

Console output:

secured_prov=1, pass_key=0

Y-BOOT 208ef13 2019-07-22 12:26:54 -0500 790da1-b-7 ROM yoda-h0-rom-16-0-gd5a8e586 FLASH:PWWWWWWAE Build $Id: git-2eebd42 $ Flash detected. flash.hw.uuid: 39483937-3207-0080-0055-ffffffffffff Bootargs: prov_every_time=1 secured_prov=1 Application Information: ------------------------ Name : Prov App Version : 1.0 Build Date : Sep 8 2023 Build Time : 13:58:53 Heap Available: 297 KB (304152 Bytes) [APP]Bootparams : -------------------- ssid= <null> passphrase= <null> dummy= <null> secured_prov =1 pass_key=0 prov_every_time=1 [APP]Bootparams end here.... [PROV]prov_is_provisioned : val = 1 [PROV] bt_gap_init [PROV] common_server_create: Inno_Provisioning InnophaseInc 1 [PROV] bt_gatt_create_service_128 ret = 0x000bf278 [PROV] Adding Characteristics [PROV] bt_gatt_add_service [PROV] bt_gap_discoverable_mode, ret = 0

secured_prov=1,pass_key=1

Y-BOOT 208ef13 2019-07-22 12:26:54 -0500 790da1-b-7 ROM yoda-h0-rom-16-0-gd5a8e586 FLASH:PWWWWWWAE Build $Id: git-2eebd42 $ Flash detected. flash.hw.uuid: 39483937-3207-0080-0055-ffffffffffff Bootargs: prov_every_time=1 secured_prov=1 pass_key=1 Application Information: ------------------------ Name : Prov App Version : 1.0 Build Date : Sep 8 2023 Build Time : 13:58:53 Heap Available: 297 KB (304152 Bytes) [APP]Bootparams : -------------------- ssid= <null> passphrase= <null> dummy= <null> secured_prov =1 pass_key=1 prov_every_time=1 [APP]Bootparams end here.... [PROV]prov_is_provisioned : val = 1 [PROV] bt_gap_init [PROV] common_server_create: Inno_Provisioning InnophaseInc 1 [PROV] bt_gatt_create_service_128 ret = 0x000bf278 [PROV] Adding Characteristics [PROV] bt_gatt_add_service [PROV] bt_gap_discoverable_mode, ret = 0

- Launch the downloaded BLE Provisioning mobile app. App icon and UI are as shown in Figure 4.

Figure 4: BLE Provisioning home screen

- To check the version of the Talaria TWO Provisioning App and other related details, user can click and open the About Us option.

Figure 5: About Us

Talaria TWO Provisioning Mobile App version is displayed as shown in Figure 6. Version varies for each release.

User has the following two options via which they can contact the InnoPhase support team.

- Contact the sales team via email: sales@innophaseiot.com

- LinkedIn, which redirects them to the InnoPhase website.

Figure 6: About Us - App version and other related details

- Click on BLE Scan if there are no services listed.

Figure 7: BLE Scan

- If the user needs to scan for unknown devices, then the highlighted toggle switch should be enabled.

Figure 8: Show/List ‘Unknown’ devices

- Click on the Inno_Provisioning service.

Figure 9: Starting Inno_Provisioning

Console output: non-secured

[121.460,022] BT connect[0]: ia:68:be:27:2e:5f:f9 aa:05:04:03:02:01:00 phy2:0/0 phyC:00 [PROV]BLE connection success [ROV] prov_srv_fn_wifi_scan prov_event_thread_entry: msg->event = 1 Creating wcm interfaceaddr e0:69:3a:00:2c:3c [PROV]140 : len = 11 [PROV] WiFi scan: Found 7 networks [PROV]168 : wc.scan_rslts_len = 75 [PROV]168 : wc.scan_rslts_len = 128 [PROV]168 : wc.scan_rslts_len = 194 [PROV]168 : wc.scan_rslts_len = 257 [PROV]168 : wc.scan_rslts_len = 322 [PROV]168 : wc.scan_rslts_len = 382 [PROV]168 : wc.scan_rslts_len = 446 [PROV]174 : len = 448 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 0 [PROV] prov_wifi_scan_rslts_send : length = 22, index=0 , rslt_len=448 [PROV]prov_wifi_scan_rslts_send: offset = 0 remaining_bytes = 448, copy_len = 18 [PROV] prov_wifi_scan_rslts_send : wc.index = 18 [PROV]prov_wifi_scan_rslts_send: index = 18, scan_rslts_len = 448 Sending {"result":[{"ssid" [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 22 [PROV] prov_wifi_scan_rslts_send : length = 22, index=18 , rslt_len=448 [PROV]prov_wifi_scan_rslts_send: offset = 22 remaining_bytes = 430, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 40 [PROV]prov_wifi_scan_rslts_send: index = 40, scan_rslts_len = 448 Sending :"InnoPhase_AE","bssid [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 44 [PROV] prov_wifi_scan_rslts_send : length = 22, index=40 , rslt_len=448 [PROV]prov_wifi_scan_rslts_send: offset = 44 remaining_bytes = 408, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 62 [PROV]prov_wifi_scan_rslts_send: index = 62, scan_rslts_len = 448 Sending ":"98:da:c4:73:b7:76", [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 66 [PROV] prov_wifi_scan_rslts_send : length = 22, index=62 , rslt_len=448 [PROV]prov_wifi_scan_rslts_send: offset = 66 remaining_bytes = 386, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 84 [PROV]prov_wifi_scan_rslts_send: index = 84, scan_rslts_len = 448 Sending "channel":11},{"ssid": [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 88 [PROV] prov_wifi_scan_rslts_send : length = 22, index=84 , rslt_len=448 [PROV]prov_wifi_scan_rslts_send: offset = 88 remaining_bytes = 364, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 106 [PROV]prov_wifi_scan_rslts_send: index = 106, scan_rslts_len = 448 Sending "","bssid":"aa:da:c4:7 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 110 [PROV] prov_wifi_scan_rslts_send : length = 22, index=106 , rslt_len=448 [PROV]prov_wifi_scan_rslts_send: offset = 110 remaining_bytes = 342, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 128 [PROV]prov_wifi_scan_rslts_send: index = 128, scan_rslts_len = 448

- For secured BLE provisioning, pairing with the device is a must. On clicking Inno_Provisioning on the mobile application, a pop-up appears with a request to pair:

Note: If the mobile application is already paired with an existing provisioning advertisement, the user will have to explicitly remove the same from paired devices before proceeding.

For unpairing a device, go to:

    - Settings → Connections → Bluetooth

    - Click on the Settings icon on the paired device → Unpair.

- secured_prov=1,pass_key=0

Figure 10: Pairing - open networks

- secured_prov=1,pass_key=1

Enter the appropriate pass_key (PIN) from the Talaria TWO console log to complete pairing.

Figure 11: passkey (PIN) from the console log

Figure 12: Pairing - with pass_key

Once the pairing is complete, the screen with a list of available APs in the vicinity opens.

- Available Wireless Network screen opens the AP listing available in the vicinity. Here, select the appropriate AP from the list.

Figure 13: Selecting the appropriate AP from the list

- Provide the passphrase for the secured AP and click on Submit. User can use the hide or show option to check the passphrase entered.

Note: If the user is connecting to an open network, they can keep the password space empty.

Figure 14: Connecting to InnoPhase_AE network by providing passphrase

- On successfully connecting to the AP, the following output is observed:

Figure 15: Successfully connected to InnoPhase network

Console output:

To connect to the configured network, “Apply and reset” is used to save the provisioned data and reset the application.

Note: If there is a need to re-provision the application, follow the procedure described in section 7.1.

Non-secure provisioning:

[PROV]prov_srv_fn_cfg_ssid:9 InnoPhase_AE [PROV]prov_conf_update: key = ssid, val = InnoPhase_AE [PROV]prov_conf_update: prov_json_object_update ret = 0 prov_srv_fn_cfg_ssid : 1 0 [PROV]prov_conf_update: key = passphrase, val = Inno@1234 [PROV]prov_conf_update: prov_json_object_update ret = 0 prov_srv_fn_cfg_passphrase : 1 1 prov_event_thread_entry: msg->event = 2 ssid = InnoPhase_AE passphrase = Inno@1234 [PROV]Checking connectivity to... ssid = InnoPhase_AE passphrase = Inno@1234 network profile created for ssid: InnoPhase [PROV] prov_srv_fn_wifi_conn_state: data = waiting[54.777,862] CONNECT:00:5f:67:cd:c5:a6 Channel:11 rssi:-54 dBm [PROV] prov_srv_fn_wifi_conn_state: data = waiting[55.512,208] MYIP 192.168.0.106 [55.512,488] IPv6 [fe80::e269:3aff:fe00:1390]-link [PROV] prov_srv_fn_wifi_conn_state: data = success [PROV] prov_srv_fn_wifi_conn_state: data = success [ROV] prov_srv_fn_cfg_apply_and_reset [PROV]prov_conf_set_provisioned 168 [PROV]prov_conf_set_provisioned 192 [PROV]prov_conf_data_apply:342 [PROV]prov_conf_data_apply:345 { "image": [ { "name": "prov_app", "version": "1.0", "start_sector": 32, "bootargs_start": 1, "ssid": "InnoPhase_AE", "passphrase": "Inno@1234", "custparam": "inno", "bootargs_end": 1 } ], "provisioned": 1, "baudrate": 2560000, "timeout": 0, "verbose": 1 } [PROV]prov_conf_data_apply:350 app_prov_cb : status = 1 Y-BOOT 208ef13 2019-07-22 12:26:54 -0500 790da1-b-7 ROM yoda-h0-rom-16-0-gd5a8e586 FLASH:P UART:SNWWWWAE Build $Id: git-34e3eddb8 $ hio.baudrate=921600 flash: Gordon ready! UART:SNWWWWWAEBuild $Id: git-34e3eddb8 $ Application Information: ------------------------ Name : Prov App Version : 1.0 Build Date : Aug 3 2022 Build Time : 03:56:17 Heap Available: 326 KB (333976 Bytes) [APP]Bootparams : -------------------- ssid= <null> passphrase= <null> dummy= <null> [APP]Bootparams end here.... [PROV]prov_is_provisioned : val = 1 [PROV]prov_is_provisioned : val = 1 ssid = InnoPhase_AE passphrase = Inno@1234 [APP] InnoPhase_AE Inno@1234addr e0:69:3a:00:13:90 app_wcm_connect:144 0x000bfa20 0x000bfa78 network profile created for ssid: InnoPhase_AE [2.340,175] CONNECT:98:da:c4:73:b7:76 Channel:11 rssi:-33 dBm [2.387,308] MYIP 192.168.0.164 [2.387,586] IPv6 [fe80::e269:3aff:fe00:1390]-link [APP]Success: Connected to InnoPhase_AE N/w [APP]------ Program Exit-------------

secured_prov=1, pass_key=0

prov_event_thread_entry[5.094,119] BT connect[0]: ia:7c:90:13:12:df:43 aa:05:04:03:02:01:00 phy2:0/0 phyC:00 [PROV]BLE connection success [ROV] prov_srv_fn_wifi_scan prov_event_thread_entry: msg->event = 1 Creating wcm interfaceaddr e0:69:3a:00:16:cc [PROV]199 : len = 11 [PROV] WiFi scan: Found 4 networks [PROV]227 : wc.scan_rslts_len = 68 [PROV]227 : wc.scan_rslts_len = 132 [PROV]227 : wc.scan_rslts_len = 199 [PROV]227 : wc.scan_rslts_len = 258 [PROV]233 : len = 260Authentication succeeded. [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 0 [PROV] prov_wifi_scan_rslts_send : length = 22, index=0 , rslt_len=260 [PROV]prov_wifi_scan_rslts_send: offset = 0 remaining_bytes = 260, copy_len = 18 [PROV] prov_wifi_scan_rslts_send : wc.index = 18 [PROV]prov_wifi_scan_rslts_send: index = 18, scan_rslts_len = 260 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 44 [PROV] prov_wifi_scan_rslts_send : length = 22, index=40 , rslt_len=260 [PROV]prov_wifi_scan_rslts_send: offset = 44 remaining_bytes = 220, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 62 [PROV]prov_wifi_scan_rslts_send: index = 62, scan_rslts_len = 260 Sending 7:2c:7c:ee:f4","channe [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 66 [PROV] prov_wifi_scan_rslts_send : length = 22, index=62 , rslt_len=260 [PROV]prov_wifi_scan_rslts_send: offset = 66 remaining_bytes = 198, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 84 [PROV]prov_wifi_scan_rslts_send: index = 84, scan_rslts_len = 260 Sending l":10},{"ssid":"dlink8 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 88 [PROV] prov_wifi_scan_rslts_send : length = 22, index=84 , rslt_len=260 [PROV]prov_wifi_scan_rslts_send: offset = 88 remaining_bytes = 176, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 106 [PROV]prov_wifi_scan_rslts_send: index = 106, scan_rslts_len = 260 Sending 25_iop","bssid":"34:0a [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 110 [PROV] prov_wifi_scan_rslts_send : length = 22, index=106 , rslt_len=260 [PROV]prov_wifi_scan_rslts_send: offset = 110 remaining_bytes = 154, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 128 [PROV]prov_wifi_scan_rslts_send: index = 128, scan_rslts_len = 260 Sending :33:91:bd:30","channel [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 132 [PROV] prov_wifi_scan_rslts_send : length = 22, index=128 , rslt_len=260 [PROV]prov_wifi_scan_rslts_send: offset = 132 remaining_bytes = 132, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 150 [PROV]prov_wifi_scan_rslts_send: index = 150, scan_rslts_len = 260 Sending ":1},{"ssid":"Asus_86U [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 154 [PROV] prov_wifi_scan_rslts_send : length = 22, index=150 , rslt_len=260 [PROV]prov_wifi_scan_rslts_send: offset = 154 remaining_bytes = 110, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 172 [PROV]prov_wifi_scan_rslts_send: index = 172, scan_rslts_len = 260 Sending _2G_iop","bssid":"24:4 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 176 [PROV] prov_wifi_scan_rslts_send : length = 22, index=172 , rslt_len=260 [PROV]prov_wifi_scan_rslts_send: offset = 176 remaining_bytes = 88, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 194 [PROV]prov_wifi_scan_rslts_send: index = 194, scan_rslts_len = 260 Sending b:fe:5e:fd:d8","channe [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 198 [PROV] prov_wifi_scan_rslts_send : length = 22, index=194 , rslt_len=260 [PROV]prov_wifi_scan_rslts_send: offset = 198 remaining_bytes = 66, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 216 [PROV]prov_wifi_scan_rslts_send: index = 216, scan_rslts_len = 260 Sending l":1},{"ssid":"Asus777 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 220 [PROV] prov_wifi_scan_rslts_send : length = 22, index=216 , rslt_len=260 [PROV]prov_wifi_scan_rslts_send: offset = 220 remaining_bytes = 44, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 238 [PROV]prov_wifi_scan_rslts_send: index = 238, scan_rslts_len = 260 Sending ","bssid":"58:11:22:71 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 242 [PROV] prov_wifi_scan_rslts_send : length = 22, index=238 , rslt_len=260 [PROV]prov_wifi_scan_rslts_send: offset = 242 remaining_bytes = 22, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 260 [PROV]prov_wifi_scan_rslts_send: index = 260, scan_rslts_len = 260 Sending :f2:a8","channel":6}]} [PROV]prov_srv_fn_cfg_ssid:12 dlink825_iop [PROV]prov_conf_update: key = ssid, val = dlink825_iop [PROV]prov_conf_update: prov_json_object_update ret = 0 prov_srv_fn_cfg_ssid : 1 0 [PROV]prov_conf_update: key = passphrase, val = InnoQA2023$ [PROV]prov_conf_update: prov_json_object_update ret = 0 prov_srv_fn_cfg_passphrase : 1 1 prov_event_thread_entry: msg->event = 2 ssid = dlink825_iop passphrase = InnoQA2023$ [PROV]Checking connectivity to... ssid = dlink825_iop passphrase = InnoQA2023$ network profile created for ssid: dlink825_iop [30.915,617] CONNECT:34:0a:33:91:bd:30 Channel:1 rssi:-32 dBm [PROV] prov_srv_fn_wifi_conn_state: data = waiting[35.658,449] MYIP 192.168.1.214 [35.658,613] IPv6 [fe80::e269:3aff:fe00:16cc]-link [35.661,211] DISCONNECTED [PROV] prov_srv_fn_wifi_conn_state: data = success [PROV] prov_srv_fn_wifi_conn_state: data = success [ROV] prov_srv_fn_cfg_apply [PROV]prov_conf_set_provisioned 173 [PROV]prov_conf_set_provisioned 197 [PROV]prov_conf_data_apply:347 [PROV]prov_conf_data_apply:350 { "image": [ { "name": "prov_app", "version": "1.0", "start_sector": 32, "bootargs_start": 1, "ssid": "dlink825_iop", "passphrase": "InnoQA2023$", "custparam": "inno", "new_boot_param_key": "new_boot_param_val", "bootargs_end": 1 }, { "name": "prov_app", "version": "1.0", "start_sector": 208, "bootargs_start": 1, "ssid": "Innophasetest", "passphrase": "123456789", "custparam": "inno", "new_boot_param_key": "new_boot_param_val", "bootargs_end": 1 } ], "provisioned": 1, "baudrate": 2560000, "timeout": 0, "verbose": 1 } [PROV]prov_conf_data_apply:355 app_prov_cb : status = 1 Y-BOOT 208ef13 2019-07-22 12:26:54 -0500 790da1-b-7 ROM yoda-h0-rom-16-0-gd5a8e586 FLASH:PNWWWWWWAEBuild $Id: git-a042e9a42 $ secured_prov=1 pass_key=0 Application Information: ------------------------ Name : Prov App Version : 1.0 Build Date : Apr 24 2023 Build Time : 20:57:23 Heap Available: 323 KB (331672 Bytes) [APP]Bootparams : -------------------- ssid= <null> passphrase= <null> secured_prov =1 pass_key=0 [APP]Bootparams end here.... [PROV]prov_is_provisioned : val = 1 [PROV]prov_is_provisioned : val = 1 ssid = dlink825_iop passphrase = InnoQA2023$ [APP] dlink825_iop InnoQA2023$addr e0:69:3a:00:16:cc network profile created for ssid: dlink825_iop [1.356,659] Trying to connect in 1 seconds [2.637,148] Trying to connect in 2 seconds [4.940,709] Trying to connect in 4 seconds [10.444,878] CONNECT:34:0a:33:91:bd:30 Channel:1 rssi:-32 dBm [11.800,951] MYIP 192.168.1.214 [11.801,114] IPv6 [fe80::e269:3aff:fe00:16cc]-link [APP]Success: Connected to dlink825_iop N/w [APP]------ Program Exit-------------

secured_prov=1,pass_key=1

prov_event_thread_entry[130.106,485] BT connect[0]: ia:57:34:ac:96:d3:11 aa:05:04:03:02:01:00 phy2:0/0 phyC:00 [PROV]BLE connection success [ROV] prov_srv_fn_wifi_scan prov_event_thread_entry: msg->event = 1 Creating wcm interfaceaddr e0:69:3a:00:16:cc [PROV]199 : len = 11 [PROV] WiFi scan: Found 10 networks [PROV]227 : wc.scan_rslts_len = 79 [PROV]227 : wc.scan_rslts_len = 137 [PROV]227 : wc.scan_rslts_len = 190 [PROV]227 : wc.scan_rslts_len = 255 [PROV]227 : wc.scan_rslts_len = 317 [PROV]227 : wc.scan_rslts_len = 378 [PROV]227 : wc.scan_rslts_len = 442 [PROV]227 : wc.scan_rslts_len = 506 [PROV]227 : wc.scan_rslts_len = 571 [PROV]227 : wc.scan_rslts_len = 630 [PROV]233 : len = 632Passkey (to be entered on remote device): 455291 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 0 [PROV] prov_wifi_scan_rslts_send : length = 22, index=0 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 0 remaining_bytes = 632, copy_len = 18 [PROV] prov_wifi_scan_rslts_send : wc.index = 18 [PROV]prov_wifi_scan_rslts_send: index = 18, scan_rslts_len = 632 Sending id":"ct_ds","bssid":"1 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 110 [PROV] prov_wifi_scan_rslts_send : length = 22, index=106 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 110 remaining_bytes = 526, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 128 [PROV]prov_wifi_scan_rslts_send: index = 128, scan_rslts_len = 632 Sending c:b7:2c:7c:ee:f4","cha [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 132 [PROV] prov_wifi_scan_rslts_send : length = 22, index=128 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 132 remaining_bytes = 504, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 150 [PROV]prov_wifi_scan_rslts_send: index = 150, scan_rslts_len = 632 Sending nnel":10},{"ssid":""," [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 154 [PROV] prov_wifi_scan_rslts_send : length = 22, index=150 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 154 remaining_bytes = 482, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 172 [PROV]prov_wifi_scan_rslts_send: index = 172, scan_rslts_len = 632 Sending bssid":"7c:10:c9:02:f2 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 176 [PROV] prov_wifi_scan_rslts_send : length = 22, index=172 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 176 remaining_bytes = 460, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 194 [PROV]prov_wifi_scan_rslts_send: index = 194, scan_rslts_len = 632 Sending id":"tplink_A91","bssi [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 286 [PROV] prov_wifi_scan_rslts_send : length = 22, index=282 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 286 remaining_bytes = 350, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 304 [PROV]prov_wifi_scan_rslts_send: index = 304, scan_rslts_len = 632 Sending d":"60:32:b1:33:b5:7b" [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 308 [PROV] prov_wifi_scan_rslts_send : length = 22, index=304 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 308 remaining_bytes = 328, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 326 [PROV]prov_wifi_scan_rslts_send: index = 326, scan_rslts_len = 632 Sending ,"channel":6},{"ssid": [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 330 [PROV] prov_wifi_scan_rslts_send : length = 22, index=326 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 330 remaining_bytes = 306, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 348 [PROV]prov_wifi_scan_rslts_send: index = 348, scan_rslts_len = 632 Sending "low_rssi","bssid":"74 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 352 [PROV] prov_wifi_scan_rslts_send : length = 22, index=348 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 352 remaining_bytes = 284, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 370 [PROV]prov_wifi_scan_rslts_send: index = 370, scan_rslts_len = 632 Sending :da:88:a6:9c:ea","chan [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 374 [PROV] prov_wifi_scan_rslts_send : length = 22, index=370 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 374 remaining_bytes = 262, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 392 [PROV]prov_wifi_scan_rslts_send: index = 392, scan_rslts_len = 632 Sending nel":11},{"ssid":"dlin [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 396 [PROV] prov_wifi_scan_rslts_send : length = 22, index=392 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 396 remaining_bytes = 240, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 414 [PROV]prov_wifi_scan_rslts_send: index = 414, scan_rslts_len = 632 Sending k650_iop","bssid":"e0: [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 418 [PROV] prov_wifi_scan_rslts_send : length = 22, index=414 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 418 remaining_bytes = 218, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 436 [PROV]prov_wifi_scan_rslts_send: index = 436, scan_rslts_len = 632 Sending 1c:fc:a3:5b:04","chann [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 440 [PROV] prov_wifi_scan_rslts_send : length = 22, index=436 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 440 remaining_bytes = 196, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 458 [PROV]prov_wifi_scan_rslts_send: index = 458, scan_rslts_len = 632 Sending el":1},{"ssid":"ASUS_O [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 462 [PROV] prov_wifi_scan_rslts_send : length = 22, index=458 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 462 remaining_bytes = 174, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 480 [PROV]prov_wifi_scan_rslts_send: index = 480, scan_rslts_len = 632 Sending utside","bssid":"58:11 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 484 [PROV] prov_wifi_scan_rslts_send : length = 22, index=480 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 484 remaining_bytes = 152, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 502 [PROV]prov_wifi_scan_rslts_send: index = 502, scan_rslts_len = 632 Sending :22:71:ee:10","channel [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 506 [PROV] prov_wifi_scan_rslts_send : length = 22, index=502 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 506 remaining_bytes = 130, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 524 [PROV]prov_wifi_scan_rslts_send: index = 524, scan_rslts_len = 632 Sending ":1},{"ssid":"asusax55 [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 528 [PROV] prov_wifi_scan_rslts_send : length = 22, index=524 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 528 remaining_bytes = 108, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 546 [PROV]prov_wifi_scan_rslts_send: index = 546, scan_rslts_len = 632 Sending u_iop","bssid":"04:42: [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 550 [PROV] prov_wifi_scan_rslts_send : length = 22, index=546 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 550 remaining_bytes = 86, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 568 [PROV]prov_wifi_scan_rslts_send: index = 568, scan_rslts_len = 632 Sending 1a:bd:6e:08","channel" [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 572 [PROV] prov_wifi_scan_rslts_send : length = 22, index=568 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 572 remaining_bytes = 64, copy_len = 22 [PROV] prov_wifi_scan_rslts_send : wc.index = 590 [PROV]prov_wifi_scan_rslts_send: index = 590, scan_rslts_len = 632 Sending :1},{"ssid":"Asus777", [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 594 [PROV] prov_wifi_scan_rslts_send : length = 22, index=590 , rslt_len=632 [PROV]prov_wifi_scan_rslts_send: offset = 594 remaining_bytes = 42, copy_len = 6 [PROV] prov_wifi_scan_rslts_send : wc.index = 596 [PROV]prov_wifi_scan_rslts_send: index = 596, scan_rslts_len = 632 Sending "channel":6}]} scan_len is greaterthan index [PROV] prov_srv_fn_wifi_scan_rslts : length = 22 [PROV] prov_srv_fn_wifi_scan_rslts : offset = 0 [PROV] prov_wifi_scan_rslts_send : length = 22, index=0 , rslt_len=0 [PROV] prov_wifi_scan_rslts_send : line = 253 Authentication succeeded. [PROV]prov_srv_fn_cfg_ssid:10 tplink_A91 [PROV]prov_conf_update: key = ssid, val = tplink_A91 [PROV]prov_conf_update: prov_json_object_update ret = 0 prov_srv_fn_cfg_ssid : 1 0 [PROV]prov_conf_update: key = passphrase, val = InnoQA2023$ [PROV]prov_conf_update: prov_json_object_update ret = 0 prov_srv_fn_cfg_passphrase : 1 1 prov_event_thread_entry: msg->event = 2 ssid = tplink_A91 passphrase = InnoQA2023$ [PROV]Checking connectivity to... ssid = tplink_A91 passphrase = InnoQA2023$ network profile created for ssid: tplink_A91 [196.123,714] CONNECT:60:32:b1:33:b5:7b Channel:6 rssi:-35 dBm [198.756,859] MYIP 192.168.1.15 [198.757,022] IPv6 [fe80::e269:3aff:fe00:16cc]-link [198.759,544] DISCONNECTED [PROV] prov_srv_fn_wifi_conn_state: data = success [PROV] prov_srv_fn_wifi_conn_state: data = success [ROV] prov_srv_fn_cfg_apply [PROV]prov_conf_set_provisioned 173 [PROV]prov_conf_set_provisioned 197 [PROV]prov_conf_data_apply:347 [PROV]prov_conf_data_apply:350 { "image": [ { "name": "prov_app", "version": "1.0", "start_sector": 32, "bootargs_start": 1, "ssid": "tplink_A91", "passphrase": "InnoQA2023$", "custparam": "inno", "new_boot_param_key": "new_boot_param_val", "bootargs_end": 1 }, { "name": "prov_app", "version": "1.0", "start_sector": 208, "bootargs_start": 1, "ssid": "Innophasetest", "passphrase": "123456789", "custparam": "inno", "new_boot_param_key": "new_boot_param_val", "bootargs_end": 1 } ], "provisioned": 1, "baudrate": 2560000, "timeout": 0, "verbose": 1 } [PROV]prov_conf_data_apply:355 app_prov_cb : status = 1 Y-BOOT 208ef13 2019-07-22 12:26:54 -0500 790da1-b-7 ROM yoda-h0-rom-16-0-gd5a8e586 FLASH:PNWWWWWWAEBuild $Id: git-a042e9a42 $ secured_prov=1 pass_key=1 Application Information: ------------------------ Name : Prov App Version : 1.0 Build Date : Apr 24 2023 Build Time : 20:57:23 Heap Available: 323 KB (331672 Bytes) [APP]Bootparams : -------------------- ssid= <null> passphrase= <null> secured_prov =1 pass_key=1 [APP]Bootparams end here.... [PROV]prov_is_provisioned : val = 1 [PROV]prov_is_provisioned : val = 1 ssid = tplink_A91 passphrase = InnoQA2023$ [APP] tplink_A91 InnoQA2023$addr e0:69:3a:00:16:cc network profile created for ssid: tplink_A91 [0.930,675] CONNECT:60:32:b1:33:b5:7b Channel:6 rssi:-30 dBm [1.007,676] MYIP 192.168.1.15 [1.007,841] IPv6 [fe80::e269:3aff:fe00:16cc]-link [APP]Success: Connected to tplink_A91 N/w [APP]------ Program Exit-------------

Errors:

- Password length is less than 8 characters:

As shown in Figure 16, if the password length is less than 8 characters, an error message pops-up with the following message: Passphrase should be >= 8 characters or EMPTY for open security.

Figure 16: Incorrect or minimum length or open security password

- Wrong password:

If user entered passphrase for secure connection is incorrect, a failure message on the Talaria TWO console screen and an alert message on the mobile app indicating the same is displayed. At this stage the user is required to reset the board or re-flash the application.

Figure 17: Alert message in case of a wrong passphrase